Analysis of the Galois Iterator					PhL		8.29.13

I added the results of these notes as best I could to the Observations section of Section 3.2 item 6.

The goal here is to interpret all stuff related to my "Galois iterator".

From Section 2.2 we had these equations

q'(x) = x q(x) - o h(x) + i  .									(2.2.4)

{q'(x)} = {x}•{q(x)} - {o}• {h(x)}  + {i}  .

{q'(x)} = {x}• {q(x)}  + {i}	 . 								(2.2.13)

α = {x}		β = {q(x)}	β' = {q'(x)}	{i} = i{1} = i 1	.			(2.2.14)

β'  =  α • β  + i1   =  α β  + i 1   .

βs+1 =  α βs +  is 1	 .									(2.2.15)

βs = αs β0   +   [ αs-1 i0  + αs-2 i1 + .... + α  is-2   +  is-1  ] 1				(2.2.16)

Discussion:  The first equation comes right from the circuit where q(x) is a polynomial whose coefficients are the register values. Thus q(x) is a remainder polynomial and its coefficients are a k-tuple representation of a GF(pk) field element. I say β = {q(x)} in the sense of abstract β corresponds to this particular remainder under division by h(x). The association α = {x} makes α be a primitive element since h(x) is a prim poly and h({x}) = h(α) = 0 which I do state somewhere. The equals sign in β = {q(x)} represents an isomorphism of some sort as I show somewhere in GA, it is not a true equality. Here is a triple association which does not yet include my matrix thing:

Then later I include the matrix

Question:  Given some q(x) remainder, what is the corresponding matrix Q ?

Answer:  This has to come from the enumeration table for GF(pk). The first powers are simple

X0		{1}   = 00001

X1		{x}    = 00010

X2 		{x2}  = 00100

etc

For example, for GF(23) I show that

so in this case we can look up any q(x) in the m-tuple A2..A0 columns and see what Xn power it matches. For example, q(x) = 101 = x2 + 1 corresponds to X6. So at least I think I know how to go back and forth between power of the matrix X and the m-tuple representation

So basically in Section 2.2 I start with a physical equation from the actual circuit which involves polys and I apply the special {...} operation to convert this to an abstract Galois field equation involving things like α and β and βs where s is just a time subscript (why didn't I use n for that?  Well, I was using n as the order of α at that time.

There are no bolded vectors in this section 2.2. There are only q(x) polynomials, but of course that does imply a vector.

From Section 3.2 I again start with the physical circuit and I quickly come up with

q'1			0	0	0	...	0	-h0		q1		i

q'2			1	0	0	...	0	-h1		q2		0

q'3			0	1	0	...	0	-h2		q3		0

q'4	            =		0	0	1	...	0	-h3	    q4	          + 	0

...				....................................................................		...		..

q'k-1		0	0	0	...	0	-hk-2		qk-1	0

q'k 			0	0	0	...	1	-hk-1		qk 		0

(3.2.2)

Now we can write this in compact vector/matrix notation as:

q' = B q + i 1										(3.2.3)

For the first time, I now have bolded vectors which represent GF(pk) elements in the k-tuple basis. And I also have the matrix B which is my primitive element. Compare

q'      =   B   q        + i 1		matrix/vector form					(3.2.3)

q'(x)    = x q(x) - o h(x) + i  .	physical equation in terms of polys			 (2.2.4)

{q'(x)}  = {x}• {q(x)}  + i{1}	remainder form						(2.2.13)

β'     =  α • β               + i1		abstract form

α = {x}		β = {q(x)}	β' = {q'(x)}	{i} = i{1} = i 1	.			(2.2.14)

What is unusual here is that in the lower two forms, all Galois elements are of the same "type":  they are all remainders, or they are all "abstract field elements". But in the first form, B is a matrix while q is a k-tuple so the field elements are on a different footing. This is just the way things come out and it is rather handy.

I want to say that in this matrix/vector form we have

B = a primitive element of GF(pk) in a matrix representation of GF(pk)

q = an element of GF(pk) in a k-tuple representation of GF(pk) associated with poly coeffs

Suppose q = (0,0...0,1,0) meaning q(x) = x. In this case we know that q = B in our equivalence equality and then we have

q'      =   B   q        + i 1

= B    B       + i  I    = B2 + i I    = some matrix element of GF(pk).

OK, I think I need to modify the Observations following (3.2.5). AT least this is the section where I need to say whatever it is I want to say.

OK, stop. I have done my best adding notes to the Observation section of scrambler doc Sec 3.2. It is just some weird mixed representation of the abstract Galois equation. I don't think there is more that one can say. At least I how have some "words" in there about this strange equation.