Transmission Line at very low Frequency			PhL		8.16.14

Reviewing this doc on 9.10.14.

1. Basic facts for traveling wave with complex k.	1

2. Apply these basic facts to a TL operating at low ω	2

3. Where does n(θ) come from?	3

4. Rewrite Section 6.5 (d) using the correct k instead of βd0 :	3

5. So how does this discussion go for low ω instead of high skin depth ω?	5

## 1. Basic facts for traveling wave with complex k.

Imagine driving an infinite transmission line, two large solid copper cylinders that go off the infinity. We drive at very low ω.  Let's assume for the moment that the network model applies in this regime, so that the traditional formula k  ≡  -j  applies. Assume also that G = 0. Then we know that

[ this is the same limit as found in the updated App Q ]

Fact 4:  In the low frequency limit with G = 0 , 							(Q.4)

Re(k)   ≈     	ω1/2 +  ω3/2

Im(k)   ≈   - 	ω1/2 +  ω3/2			ω << R/L

Now put that above aside for a moment, and consider a wave going down the line with complex k. We then assume our usual form ej(ωt-kz) and we find that

exp[ j(ωt-{kr+jki}z]  =  exp[+kiz] exp[ j(ωt-krz)]

Therefore, in talking about a phase front, only kr plays a role, as I have long suspected. The phase velocity is determined in this manner:

ωt-krz = constant

Apply dt to get

ω = kr(dz/dt)		=>		dz/dt = ω/kr  ≡  vφ  = velocity of phase fronts

Meanwhile, the wavelength of a wave is determined from

λ = 2π/kr

Here kr is the number of radians of wave phase per meter [ since phase = ωt-krz = constant], so that 1/kr is the meters per radian of phase. The number of meters per full wave is then 2π times this latter.

Finally, the decay distance [ that is, the 1/e dropoff distance ] is Δz = 1/(-ki).

We now summarize these facts:

Traveling Wave with Complex k

vφ = ω/kr = velocity of phase front going down the line

λ = 2π/kr = distance between fronts of the same phase = wavelength

Δz =  1/(-ki) = decay distance for 1/e decay of the wave

[ I think everything in the box is correct ]

## 2. Apply these basic facts to a TL operating at low ω

Now having determined the above for general complex k, we apply it to our transmission line k where

kr =   ω1/2

ki=  - ω1/2

We then find that

vφ = ω/kr = ω / [ω1/2] = ω1/2			phase velocity

λ = 2π/kr = 2π / [ω1/2] = 2π ω-1/2			wavelength

Δz = 1/(-ki) = 1/kr = ω-1/2					decay distance

As ω → 0, we find that

vφ  → 0  as 		phase velocity goes to zero, an unexpected result

λ → ∞  as 1/		wavelength goes to ∞, an expected result

Δz  → ∞			decay distance goes to ∞, expected result since know I→ 0

The feature unexpected by me is that the phase velocity slows down to 0 ! This should have some implications below.

Now I claim that

V(z) = V(0) ej(ωt-kz)

q(z) = q(0) ej(ωt-kz)

q(z) = C V(z)

So the driving voltage source V(0) determines q(0), and so we know q(z) going down the line. This in turn determines n(θ,z) going down the line, from the usual 2D electrostatic solution.

[ everything in Section 2 above seems correct ]

## 3. Where does n(θ) come from?

Now we come to a big question:  In this low frequency regime, "where does n(θ) come from? "

I argued in Section D.9 (c), and I quote,

"The first hypothesis might be that the individual electrons which make up n(θ) simply travel at vd in the z direction down the conductor surface, and n(θ) is not fed by any radial currents inside the conductor. In this case one would have KzD(θ) = vd n(θ). But we know this is not what happens. Apart from the massive energy required to achieve relativistic electron velocities, we know from Appendix N.1 that the electrons in the Debye layer in fact drift along at something like ~ 1 mm/sec, just as do the regular conduction electrons in the conductor bulk. "

Now in our TL very low ω limit, recall that vφ → 0 and so q(z) moves very slowly in the z direction. in this case, both arguments made above are invalid. Suppose vφ = .01 mm/sec. Then the Debye layer drift could easily supply that flow. Also, no relativistic issues. And of course the bulk can also carry charges at this slow rate.

[ the linear charge density q(z) appears to move at phase velocity vφ just as do all other transmission line quantities. This does not mean any actual physical charged particles are moving at vφ .]

Comment:  This may be the Breakthrough I have been long looking for!![ hah!] In the ω→0 limit, if the surface charge is supplied by the Debye or bulk layer, then you have to include this in the boundary condition, and I do NOT include this because I argued that all charge came from charge pumping by Jr. So maybe my BC is only valid when vφ is very large.

[ current flow in the bulk only gets to the surface via Jr, and that is why Jr appears in n(θ). But current flowing along the surface is already there, at the surface, so this would  be a Jz Debye current ]

I guess I will now go off and rethink my section on Debye surface currents!  [ see separate doc ]

[ I have already reviewed that separate doc, it was not enlightening ]

## 4. Rewrite Section 6.5 (d) using the correct k instead of βd0 :

[ I am going to read  the existing lines doc Section 6.5 right now, 9.10.14:

(a) seems OK

(b) I correctly claim β = β' at large ω ("low loss").  This section seems all OK

(c) OK, but I am always talking large ω where k = βd0.

(d) I still assume "skin effect regime" so large ω so k = βd0. But perhaps the point made in the following rewrite is that I could make this section apply for general k and not just for βd0. I start with (6.5.14) which is presumably valid for all ω , and same for (6.5.15). But THEN I go to skin effect limit on next few equations. OK , I just now edited the lines doc version so it looks more like the stuff right below, but in the end we really have k = βd0 so it does not matter much.

#### (d) The relationship between Jz(a,θ) and n(θ) obtained from div E = 0

[ see notes just above ]

Here, assuming the skin effect regime and making a few assumptions, we obtain (6.5.13) directly from the div E = 0 equation and the charge pumping boundary condition, just to provide some intuition about the linkage between Jz and n(θ).

The charge pumping boundary condition of Appendix D says (r = a means r = a-ε)

Er(r=a,θ) = (jω/σ) n(θ) 						(D.2.24)		(6.5.14)

so the pattern of n(θ) is directly mapped to Er(r=a,θ) at the surface. But we are interested in Ez since our current density of interest is Jz = σEz. The condition div E = 0 in cylindrical coordinates reads,

∂r (r Er(r,θ,z)) + ∂θEθ(r,θ,z) + r ∂zEz(r,θ,z) = 0 .

For r = a we know from (3.7.0) that Eθ(a,θ) = 0 and ∂θEθ(r,θ) = 0, so for r just below the surface we expect

∂r (r Er(r,θ,z)) + r ∂zEz(r,θ,z)  ≈ 0		// near r = a

∂r (r Er(r,θ)) - jk r Ez(r,θ)	 ≈ 0	     // using ∂z → -jk, see (D.1.16), then cancel ejkz factors

Ez(r,θ)  ≈  (1/jk) (1/r) ∂r (r Er(r,θ))  .	// near r = a					(6.5.15)

For the symmetric-environment round wire, we know from (2.2.29) that

Ez(r) = Ez(a)  .									(2.2.29)

Taking the large argument limits of the two Bessel functions using (2.3.3) and (2.3.6), we find that in the skin effect regime,

Ez(r,θ) ≈ Ez(a,θ) e(r-a)/δ  ej(r-a)/δ							(6.5.16)

which we note has the same general form as the simple result (2.1.8) with x = a-r which is e-x/δ e-jx/δ and is also consistent with (2.3.7) for magnitude. If we blindly assume this same equation applies to Er(r,θ) and Er(a,θ), then

Er(r,θ) ≈ Er(a,θ) e(r-a)/δ  ej(r-a)/δ	  .						(6.5.17)

Then (6.5.15) says,

Ez(a,θ)  =  (1/jk) Er(a,θ)[ (1/r) ∂r (r e(r-a)/δ ej(r-a)/δ) ] |r=a

= (1/jak) Er(a,θ) [1/2 + (1+j)(a/δ) ] 							(6.5.18)

where the derivative is done by Maple,

If in (6.5.18) we use the boundary condition (6.5.14) that Er(a,θ) = (jω/σ) n(θ), the result is

Ez(a,θ)  =  (1/jak) {(jω/σ) n(θ)} [1/2 + (1+j)a/δ ]

= (1/jak) {(jω/σ) n(θ)} (1+j)a/δ ]		// ignore 1/2 relative to a/δ

= (1/jk) {(ω/σ) n(θ)} (j-1)/ δ ]

= (1/jk) {(ω/σ) n(θ)} β ]			// (6.5.8)

=  (-jω/σ) (β/k) n(θ)  .								(6.5.19)

Putting this into (6.5.16) then gives

Ez(r,θ) ≈ (-jω/σ) (β/k) n(θ) e(r-a)/δ  ej(r-a)/δ 					(6.5.20)

which agrees with our earlier result (6.5.13) quoted from Appendix D. Although we just guessed at the form (6.5.17), that form is verified in box (D.10.13). The bottom line here is that Ez (and thus Jz) "tracks" n(θ) for its θ dependence.

[review OK to here ]

## 5. So how does this discussion go for low ω instead of high skin depth ω?

We certainly get to this early step, which comes just from div E = 0 inside the conductor,

Ez(r,θ)  ≈  (1/jak) ∂r (r Er(r,θ))  .		// near r = a					(6.5.15)

This is my little "linkage" between Er and Ez. This is only valid below the Debye layer, by the way, since otherwise there is some free charge to worry about and then div E = ρ/ε0.  Another concern inside the Debye layer is that Ohm's Law is replaced by J = σE - D grad ρ  where again ρ is that free charge, and this would imply that Jr ≠ σEr in the Debye layer.

So let's assume we are close to r = a, but not so close that we are in the Debye layer. Then the above linkage ought to be valid, and is also valid for current densities,

Jz(r,θ)  ≈  (1/jak) ∂r (r Jr(r,θ))

The CPBC says (here for fun I include the Debye term)

jω n(θ)  =  Jr(a- λD,θ)   + jk λd Jz(a,θ)

At low ω, not only is λd tiny, being 10-10 m, but k is also small, since

Re(k)   ≈     	ω1/2

Im(k)   ≈   - 	ω1/2			[ correct for G = 0 and new App Q or old App Q ]

[ So I have a solid arm-waving argument (λd and k both tiny as ω→0) why Debye does not matter ]

It is hard for me to imagine that the Debye term shown above is significant, but somehow it might be. So let's ignore the Debye term for now and write

jω n(θ)  =  Jr(a,θ)

where r = a is the same location appearing in the linkage equation above. So we now have:

Jz(r,θ)  ≈  (1/jak) ∂r(r Jr(r,θ))		div E = 0

Jr(a,θ)  = jω n(θ)				CPBC

Now assume that n(θ) has a monstrous huge peak at θ = π because our two cylinders are almost touching each other!  Recall n(θ) plots from bipolar doc showing such peaks. The CPBC certainly says that, for any ω > 0,

Fact 1:  Jr(a,θ) is a very strong function of θ going around the cylinder perimeter.

Now since the world is a smooth place, we can presume I think that

Fact 2:  Jr(r,θ) is a very strong function of θ going around the cylinder perimeter for r ≤ a but close to a.

By strong function in the above two Facts, I mean that the scale of the function depends strongly on n(θ) so that the function Jr(r,θ) has a strong peak at θ = π. Where n(θ) is very large, we need lots of radial pumping to provide the surface charge density!

Now we take this strong function of θ and we have

∂r (r Jr(r,θ))  = Jr(a,θ) + a [∂rJr(r,θ) ]|r=a

This is the sum of two terms. The first term has a very strong peak at θ = π. We are not sure what the second term is doing. It just seems exceedingly likely that the second term also has a strong peak at θ = π. Since we must have Jr(0,θ) = 0 on the center line, it seems likely that Jr increases monotonically to its value on the surface and thus ∂rJr will be positive, so we add to the first term. This is a generic argument. We can of course peek at the solution of Appendix D which is this, for small ω,

Jr(r,θ)  =  (j/2) I σ Rdc (ak) {(r/a) +  Σm=1∞ [(r/a)m+1 +  (r/a)m-1] ηm cos(mθ) }	(D.11.8)

Jr(a,θ)  =  (j/2) I σ Rdc (ak) {1 +  2 Σm=1∞ ηm cos(mθ) }

∂r Jr(r,θ)  =  (j/2) I σ Rdc (ak) {(1/a) +  (1/a)Σm=1∞ [(m+1)(r/a)m +  (m-1)(r/a)m-2] ηm cos(mθ) }

a [∂rJr(r,θ) ]|r=a  =   (j/2) I σ Rdc k  {1+  Σm=1∞ [(m+1) +  (m-1)] ηm cos(mθ) }

=   (j/2) I σ Rdc (ak)  {1 +  2 Σm=1∞ m ηm cos(mθ) }		ηm = (-1)m e-m|ξ|

Both functions Jr(a,θ) and a [∂rJr(r,θ) ]|r=a have strong  positive peaks at θ = π since there

Jr(a,π)                =  (j/2) I σ Rdc (ak) {1 +  2 Σm=1∞ e-m|ξ| }

a [∂rJr(r,π) ]|r=a  =  (j/2) I σ Rdc (ak)  {1 +  2 Σm=1∞ m e-m|ξ|  }

and if anything the second peak is stronger.

My conclusion is this:

Fact 3:  ∂r(r Jr(r,θ)) is strongly peaked at θ = π.

Therefore, I conclude that, from my linkage condition near the surface,

Fact 4:  Jz(r,θ) is strongly peaked at θ = π, since our linkage above says,

Jz(a,θ)  ≈  (1/jak) ∂r (r Jr(r,θ))

This then says that Jz(a,θ) is highly asymmetric around the perimeter of the cylinder. From that, I would conclude that Jz(r,θ) is highly asymmetric all across the cross section. This leads to

This conclusion applies for our infinite transmission line and shows that Jz stays asymmetric right to the limit ω→ 0.  The conclusion does not directly apply to shorted cylinders since ∂z → jk does not apply in that case, so one has then to implement the "reflection scenario" in order to get a conclusion for that situation. This process of course injects new uncertainty.

[ I agree with all these conclusions -- my theory without Debye in BC says Jz is asymmetric at low ω, and I give a good argument above for why Debye should be ignored especially as ω→ 0. ]