Reflection Scenario v 2						PhL		6.20.14

The reflection model here is improved over last time, but it does not kill off the Jz asym at low ω, see item 4 General Comments below. Far below is an early proof that E tracks n(θ). Then I am off trying to blame it all on the radial Hall effect, but then somehow that idea collapsed. I eventually concluded that radial Hall is too small an effect to cause Jz asym. (10.8.14)

Here I quote details figured out on 6.19.14 in "reflection scenario", but starting deep into the document at a place titled "Resume this development on 6.19.14"  which is on page 16.

[ When I wrote this, I was imagining "gold" to be a perfect conductor. I later learned that it is a worse conductor than copper, but I leave it as gold here with the idea of perfect conductor. ]

[ Quantity βd is now called k in Appendix D]

[ no loss because the reflected wave has same amplitude as the incident wave ]

1. We imagine a "gold block" sitting at z = L which causes the reflection. The basic solution superposition is this:

Er(r,m,z,t)  = (j/4) ηm I Rdc (aβd) gm   {  ej(ωt-β[z-L]) - ej(ωt+β[z-L]) }

= (j/4) ηm I Rdc (aβd) gm { 2j sin(-βd[z-L]) } ej(ωt)

So get rid of the time to define

Er(r,m,z)  =(j/4) ηm I Rdc (aβd) gm { 2j sin(-βd[z-L]) }  .

So then we have

Ez(r,m,z)  = (1/4) ηm I Rdc (aβ') fm { 2 cos(-βd[z-L]) }

Er(r,m,z)  = (j/4) ηm I Rdc (aβd) gm { 2j sin(-βd[z-L]) }

Eθ(r,m,z)  = (1/4) ηm I Rdc (aβd) hm { 2j sin(-βd[z-L]) } .

The justification is as follows:

Argument #1: since Er and Eθ are transverse to the gold block, and since they are 0 inside the gold block, and since Et must be continuous, we need then to have the sine form.

Argument #2: The left going wave has βd → -βd and this is why I show the minus sign between the terms in Er(r,m,z,t) above.  [ ??? ]

Fact:  Looking at Ez(r,m,z) above, we have asym Jz and this is in the model right from the very start. So nothing I do below is going to make asym Jz "go away" at ω = 0. Recall that for ω→0.

fm  = (r/a)m (m+1) (2/β'a)	f0 =  4/(aβ')

and therefore

Jz(r,m,z)  = σ (1/4) ηm I Rdc (r/a)m (m+1) (2{ 2 cos(-βd[z-L]) }		m ≠  0

Note added 8/6/14 in review:

=  =    =  (1/2) ηm (r/a)m (m+1)		m ≠0

and this shows that the m ≠ 0 moments never go away. This does assume β'a is very small.

2. Using  Jz = σEz with Ezas above, it is possible to compute Itot(z), the total current in the wire. I do that in the original document and I will summarize that calculation here.

Jz(r,m,z)  = σ (1/2) ηm I Rdc (aβ') fm  cos(βd[z-L])

Jz(r,θ,z) = Jz(r,m=0,z) + 2Σm=1∞ Jz(r,m,z) cos(mθ)

The last terms give nothing in the Itot integration so we then have

Itot(z) = ∫dA  Jz(r,m=0,z)   = 2π  !Syntax Error, Idr r Jz(r,m=0,z)

=  2π  !Syntax Error, Idr r  [σ (1/2) I Rdc (aβ') f0(r)  cos(βd[z-L]) ]

= 2π [σ (1/2) I Rdc (aβ') cos(βd[z-L]) !Syntax Error, Idr r f0(r)

But then as I show in detail in the original doc,

!Syntax Error, Idr r f0(r) =  2 a/β'

so then

Itot(z) = 2π [σ (1/2) I Rdc (aβ') cos(βd[z-L])  2 a/β'

= 2π [σ (1/2) I 1/(πa2σ) (a) cos(βd[z-L])  2 a

= 2 I  cos(βd[z-L])

So this tells us the current at any point in the shorted cylinder line, and the current is max when z = L where it has the value 2I where I is the parameter which appears in the App D solutions. Notice that I is no longer the same as i(0) because i(z) is now Itot(z). As expected, we have Itot(z) being a standing wave.

If we use the no-loss βd, we find that at ω = 0 βd = 0 and then

Itot(z, ω=0)  = 2I

3. I think the right model for n(θ) for the reflection superposition is this

ntot(θ,z,t) = n(θ) [  ej(ωt-β[z-L]) - ej(ωt+β[z-L]) ]

= n(θ) 2j sin(-βd(z-L))ejωt

ntot(θ,z)   = n1(θ) 2j sin(-βd(z-L))

where n(θ) is the single-wave n(θ) as computed in Chapter 6 for the two round wires. The total charge then per unit length is going to be

qtot(z) = 2j sin(-βd(z-L)) ∫dθ a n(θ)  =   2j sin(-βd(z-L)) q(0)

where q(z=0) is the single-wave charge/length. Since the capacitance C is fixed by geometry regardless of what waves are going on, we conclude that

Vtot(z) = qtot(z)/C   =   2j sin(-βd(z-L)) q(0)/C   =  2j sin(-βd(z-L)) V(0)

where V(0) was called just V in appendix D for the single wave. So let's use this form

Vtot(z) =   2j sin(-βd(z-L)) q(0)/C

as a description of Vtot(z) on our reflecting waves line! It vanishes at z = L as expected, and it is a standing wave.

We can then define the impedance at any point on the line to be

Ztot(z) =  Vtot(z)/Itot(z) =  2j sin(-βd(z-L)) q(0)/C    /    2 I  cos(βd[z-L])

= - 2j q(0)/(2IC)tan(βd[z-L])   =   q(0)/(jIC)tan(βd[z-L])

and this has a familiar look from ancient times, it has zeros and poles going along the line.

At the left end of our line we have: [ first argument is z ]

Itot(0;ω)  =  2 I  cos(βdL)

Vtot(0;ω) =   2j sin(βdL) q(0)/C		// this is the "driving voltage" for the reflection problem

Ztot(0;ω)  =  q(0)/(jIC)tan(βd[-L])   =  [ j q(0) / IC]  tan(βdL)

If we assume that βd → 0 as ω→0, then in the DC limit with no-loss assumed we get

Itot(0;0)  =  2 I

Vtot(0;0) =  0		// this is the "driving voltage" for the reflection problem

Ztot(0;0)  =  0

We get Vtot(0;ω) =  0  because this is a perfect line with no loss! So perhaps better to use our loss model version of βd which says βd = (ω-jωc)/vd.Then our limit is βd → -jωc/vd which recall is a small quantity. Then

cos(βdL)  →  cos(-jωc/vd L)   ≈  1		since L is not going to be light seconds long.

sin(βdL) → sin(-jωc/vd L)  ≈  -jωc/vd L  since sinx = x

tan(βdL)  →   -jωc/vd L  as well.

Then we find that

Itot(0;0)  =  2 I

Vtot(0;0) =   2j sin(βdL) q(0)/C  = 2j [-jωc/vd L] q(0)/C  = 2 [(ωc/vd) L] [q(0)/C ]

Ztot(0;0)  =  [ j q(0) / IC]  tan(βdL)  =  [ j q(0) / IC] [  -jωc/vd L]  =  [ q(0) / IC] [ ωc/vd L]

So now at least we have some kind of DC limit for these quantities.

If I now force Vtot(0;0) to be some kind of Vdrive (perhaps from a low int imp generator), that gives a value for q(0) from the middle line above. Recall that I was the single-wave current. In that single wave scenario we had

i(z) = q(z) v											  (4.11.19a)

So I then argue that q(0) / I  = 1/vd and then I can restate a lot of results above

Vtot(z;ω) =   - j 2I [1/(Cvd)]  sin(βd(z-L))

Itot(z; ω) =       2I  cos(βd[z-L])

Ztot(z;ω) =   -j [1/(Cvd)] * tan(βd[z-L])		dimensions good!

Vtot(0;ω) =    j 2I [1/(Cvd)]  sin(βdL)		//  = Vdrive

Itot(0; ω) =      2I  cos(βdL)

Ztot(0;ω) =   j [1/(Cvd)] * tan(βdL)

Vtot(0;0) =    j 2I [1/(Cvd)] [-jωc/vd L]  = ( 2I/C) [ωc/vd2] L

Itot(0; 0) =      2I

Ztot(0;0) =    [1/(C)]  [ωc/vd2] L

Now in the loss model we have

ωc  =   (RDCC) (vd2/2)		=>	ωc/vd2 =  (RDCC)/2

so we can rewrite

Vtot(0;0) =  ( 2I/C) [(RDCC)/2] L  =  ( I) [(RDC)] L  = I / (LRDC)

Itot(0; 0) =      2I

Ztot(0;0) =    [1/(C)]  [(RDCC)/2] L  =   [(RDC)/2] L  = (LRDC/2)

where recall that RDC is the sum of both wires. So these last results seem reasonable, not sure of factors of 2. BUT, we still have asym JzI am pretty sure!

4.  General Comments

The superposed reflection solution is perhaps accurate to describe high ω action on such a shorted line with two cylinders. However, the Jz distribution remains asymmetric all the way down to ω = 0 and we know this result is incorrect. This tells us that something in the theory is not accurate at low frequencies.

I have always felt that everything in Appendix D was accurate down to DC. So I think the solutions obtained there for E are all good down to ω = 0. I assume as ansatz a wave form and I use the two BC's and I get all those solutions one sees there in various places.

Since Appendix D is correct down to ω = 0, where does the problem lie??  I do think that n(θ) on the two cylinder line is accurate down to DC.  I do think the CPBC is valid all the way down as well.

Now it is true that the Chapter 4 transmission line theory is NOT valid down to ω = 0. In particular, Az is not constant on the cylinders. The fields E and B are not at right angles. So something strange happens out in that dielectric.  [ I think it is T(z) which makes e-jkz not work at low ω, but also the transverse Az equation is invalid as stated since Az not constant on the surface for low ω. 10.8.14 ]

In our reflection model, we do at least get Itot(z; ω)  → a DC current 2I, so this DC result at least is what I want to see. But the issue is the non-uniformity of the current!!! The Appendix D equations say Jz is non-uniform, and I am saying that the App D equations are good down to DC!  (confirmed once again below).

I have a very loose bolt somewhere, and I simply cannot find it, despite 4 months of looking for it maybe 10 hours a day! The "reflection scenario" seems to me to be a completely viable model of the shorted two cylinders, but it gives the wrong ω = 0 limit. Maybe I can make a list of possible locations of the error.

1. The expo ansatz at the start of App D is no good at low ω because it predicts the wrong result even though that result satisfies all Max and Helm equations and the two BC's. [ well, the ansatz is no good at low ω because the TL equations are no good there. ]

2. Something I am overlooking in App D restricts App D results to be invalid at low ω.

3. Something is just wrong with the "reflection scenario" as a model for the shorted cylinders.

4. I have a math error in App D so E fields I get are in fact wrong, through they satisfy Max and Helm.

5. Somehow n(θ) goes away at DC, the surface charge "runs away" or what have you.

6. In the CPBC which says Er(r=a,m) = (jω/σ) Nm, at ω = 0 this BC stops doing anything, and says Er(r=a,m) = 0, so the mechanism causing asymmetry goes away. Addressed in 6.5 (d).

************************************

I have said that losses build up at low ω, but have not made this too clear. Certainly below ωc we have no wave and all exponential decay!  So I am getting something like this for a solution

Ez(r,m,z)  = (1/4) ηm I Rdc (aβ') fm { 2 cos(βd[z-L]) }

But what does this mean when βd ≈  =- jωc/vd . Then the above says

Ez(r,m,z)  = (1/4) ηm I Rdc (aβ') fm { 2 cosh(ωc/vd[z-L]) }

So yes, there is expo increase going backwards from z = L to z = 0. But maybe it is better to view the sum of waves again

{  ej(ωt-β[z-L]) - ej(ωt+β[z-L]) }

I think suddenly that this might be wrong in the lossy situation. The right-moving wave decays before it hits the gold wall, and so the amplitude of the reflection is not unity after all. So at low ω, we are trying to deal with strongly decaying waves, and I guess the theory above no longer works in this case.

The odd thing is to just consider the right-going wave with its basic ej(ωt-βz) . At low ω, this has a strong amplitude decay, though that depends on (ωc/vd)L. It might NOT be such strong decay. But you have to remember that the DC situation is "all loss".  The Belden example had a decay distance of a few km, so in fact decay is NOT rapid.

Question:  I like to write i(z) = i(0) ej(-βz) .  But how can the current decay in the ω = 0 resistor where the current has to be the same constant value at all z??  So this form for i(z) seems very wrong near ω = 0. Where did this equation come from??   It first appears in (4.7.3) and I state the decay in (4.9.2) . Well the loss involves RdcC and this product stays the same all the way to ω → 0. You still get loss because you still have some C left. But at absolute ω = 0, the C can have no effect.

Still, I think this is a significant question. Whatever leads to the above equation has something wrong as you approach ω → 0 since we MUST find in that limit that i(z) = constant.

Question:  As you go to the ω=0 limit, we expect to see V(z) "drop off" as we go down the line, but we don't expect to see i(z) drop off. They are different in this limit, and somehow that difference is getting lost. If I say that Ei all drop off, then Jz has to drop off, and then i(z) drops off.

***************************

Idea done before:  Suppose you take Appendix D solutions in the two directions without evaluating the constants. Then you get

EzR(r,m) =   - j (β'/βd)  Jm(x)

EzL(r,m) =   - j (β'/βd)  Jm(x)

In order to get a standing wave you are forced to having the two K's equal!!!

Same for the a coefficients. So then we can say

Ez(r,m,z)  = [- j (β'/βd)  Jm(x)] { 2 cos(-βd[z-L]) }

Er(r,m,z)  = [am x-1 Jm(x) +  Jm+1(x)]{ 2j sin(-βd[z-L]) }

jEθ(r,m,z)  = [ - am x-1 Jm(x) + ( +  ) Jm+1(x) ] { 2j sin(-βd[z-L]) } .

n(θ,z)  = n(θ) { 2j sin(-βd(z-L)) }

Nm(z)  = Nm  { 2j sin(-βd(z-L)) }

NOW let's apply the two BC's. The Eθ one requires that

[ - am xa-1 Jm(xa) + ( +  ) Jm+1(xa) ]  = 0			// just as in (2) of lines p 298

The CPBC must hold at each value of z,

Er(r=a,m,z) = (jω/σ) Nm(z)

or

[am xa-1 Jm(xa) +  Jm+1(xa)]{ 2j sin(-βd[z-L]) } =   (jω/σ) Nm  { 2j sin(-βd(z-L)) }

or

[am xa-1 Jm(xa) +  Jm+1(xa)] =   (jω/σ) Nm				// just as in (1) of lines page 298

So the K and a are going to come out exactly the same, although now I am applying it to the total superposition wave.

So try as I might, I cannot blame the "failure" of the reflection scenario on a bad CPBC.

************************************************************

The div E tracking proof at low frequencies.

Start off as in (6.5.14) and near the surface we get

Ez(r,θ)  ≈  (1/jβd) (1/r) ∂r (r Er(r,θ))	  .	// near r = a					(6.5.15)

Ez(a,θ)  ≈  (1/jβd) (1/a) [∂r (r Er(r,θ))]r=a-	  .	// near r = a		STARE		(6.5.15)

So this says probably that Ez is asym if Er is asym. But, the CPBP says

Er(r=a,θ) = (jω/σ) n(θ) 						(D.2.24)		(6.5.14)

This clearly says that, as ω → 0, although n(θ) maintains its shape, we get Er(r=a,θ) → 0. Since my expected limit is that Er(r,θ)  = 0 every where inside at ω = 0, certainly derivatives are 0. Then the above line says that Ez(a,θ) → 0, assuming we use the loss model for βd.  But I expect to have Ez = come constant uniformly inside the wire, not Ez= 0. So what is going wrong here? Recall that ansatz says:

Ez(r,θ,z) = Ez(r,θ) ej(ωt-β[z-L])

With the loss model, we get βd → -jωc/vd  = a constant as ω→0. Therefore, Ez(r,θ,z) = constant is RULED OUT by the ansatz, right at the start!  So it is impossible to obtain Ez = constant in any limit of Appendix D results !!!  Since Ez = constant is the correct ω = 0 limit, Appendix D cannot take us there because it assumes at the start that Ez = constant is ruled out.

Wrong! My issue is not constant in z, it is constant in θ. Well, go back and stare some more:

Ez(a,θ)  ≈  (1/jβd) (1/a) [∂r (r Er(r,θ))]r=a-	  .	// near r = a		STARE		(6.5.15)

Again, as ω→ 0 we seem to get Er(a,θ) = 0 just below the surface from the CPBC. This seems to be causing Ez(a,θ) → 0.  This is the issue. This is the answer that is wrong. The true result has Ez(a,θ) → Ez0(z) which we allow as may decay in z. But the above is saying Ez(a,θ) → 0. I think this is the core of my problem somehow. For the infinite line, we really do have Ez→ 0. But my issue now is with the reflection scenario, where we still have div E = 0 just below the round wire surface.

June 21, 2014

I realized last night that, amazingly enough, it is the "radial Hall effect" that is going to resolve the little paradox I state above. The conclusions of the radial Hall section are these:

1) At DC, inside the wire there is a radial field Er(r,θ) = Es(r/a) where Es is a constant. However, there is no radial current Jr that goes along with it. Thus, we have Er(r,θ) = Es(r/a) with Jr(r,θ) = 0. The normal Ohm's Law is thus violated as outlined in App N.

2) The CPBC really says this

Jr(r=a,θ) = (jω) n(θ)

since it comes from div J = -∂tρ. The CPBC is about Jr, and not about Er. Thus, we ω→ 0, we end up with Jr(a,θ) = 0 which is consistent with the radial Hall effect result.

3) The paradox noted above had this form

Ez(a,θ)  ≈  (1/jβd) (1/a) [∂r (r Er(r,θ))]r=a-	  .	// near r = a		STARE		(6.5.15)

My paradox was that I felt Er → 0 from the CPBC as ω→0 and thus Ez → 0, but I know Ez ≠ 0 .

To resolve this paradox, we have to "back up" and re-examine div E = 0 near (6.5.15). First of all, we have to set div E = ρ/ε0 where ρ is the constant Hall free charge. Then doing everything else the same we get

∂r (r Er(r,θ,z)) + r ∂zEz(r,θ,z)  ≈ ρ/ε0		// near r = a

∂r (r Er(r,θ)) - jβd r Ez(r,θ)	≈ ρ/ε0	     // using ∂z → -jβd, see (D.1.16), then cancel ejβz factors

jβd r Ez(r,θ)	  ≈  ∂r (r Er(r,θ)) - ρ/ε0

Ez(r,θ)   ≈  (1/jβdr) ∂r (r Er(r,θ)) -  (1/jβdr)ρ/ε0

But that paradox hopefully goes away when we write Jr → 0 from the CPBC and Er(r,θ) = Es(r/a). Then we get

Ez(a,θ)1st term  ≈   (1/jβda) [∂r (r Er(r,θ))]r=a-  = (1/jβda) [∂r (r Es(r/a)]r=a-

=  (1/jβda) (Es/a) [∂r r2]r=a-  = (1/jβda) (Es/a) 2a

=  (Es/a) (1/jβd) 2  = 2Es (1/jaβd)

The first term is a constant, and to this we now add the second term where

ρ  =  ε0(2Es/a)  .										(N.7.16)

So with both terms we get

Ez(a,θ)  ≈  (1/jβdr) ∂r (r Er(r,θ)) -  (1/jβdr)ρ/ε0

=  2Es (1/jaβd)  -  (1/jβda) 1/ε0[ε0(2Es/a)]

=  (1/jaβd) [ 2Es - 2Es] = 0

oops! Something went wrong. In Appendix N I compute ρ from divE but I set  ∂zEz = 0 since the wire is infinitely long and uniform.  But in Chap (6.5) I have ∂zEz = -jβdEz ≠ 0. This latter expression allows for slow expo decay of Ez in the TL if I use my little loss model. But at ω = 0, the capacitance has no effect in the TL and I have G = 0 all the time, so then I expect Ez NOT to decay. On the other hand, I know that for ω = 0 in this case, we really do have Ez = 0 because there is no current for the ∞TL. So maybe my oops is OK for the single wave scenario.

Let's try the above for the reflection scenario. We then have

∂r (r Er(r,θ,z)) + r ∂zEz(r,θ,z)  ≈ ρ/ε0		// near r = a

But now

Er(r,m,z)  = (j/4) ηm I Rdc (aβd) gm { 2j sin(-βd[z-L]) }

Ez(r,m,z)  = (1/4) ηm I Rdc (aβ') fm { 2 cos(-βd[z-L]) }

Just writing this out gives

(j/4) ηm I Rdc (aβd) { 2j sin(-βd[z-L]) }∂r (r gm(r))

+ r ∂z (1/4) ηm I Rdc (aβ') fm { 2 cos(-βd[z-L]) }    ≈ ρ/ε0

In this case, we have

∂z{ 2 cos(-βd[z-L]) = {-2 βd sin( βd[z-L]) }

and then the equation reads

(j/4) ηm I Rdc (aβd) { 2j sin(-βd[z-L]) }∂r (r gm(r))

+  a (1/4) ηm I Rdc (aβ') fm {-2 βd sin( βd[z-L]) }    ≈ ρ/ε0

or

- (1/4) ηm I Rdc (aβd) { 2sin(-βd[z-L]) }∂r (r gm(r))

-   a (1/4) ηm I Rdc (aβ') fm {2 βd sin( βd[z-L]) }    ≈ ρ/ε0

or

- (1/4) ηm I Rdc { 2sin(-βd[z-L]) } [ 	(aβd) ∂r (r gm(r)) + (aβ') fm ]  ≈  ρ/ε0

Well, let's pause on this for more comments.

Comment 1.  In Appendix D, I use βd in the opening ansatz. This is really only βd in the lossless case, so the name βd is a misnomer really. But going ahead with this symbol βd and its ansatz, we grind through Appendix D and we get the answers we get there as functions of βd. This analysis ignores the possibility of a tiny Hall charge density ρ inside the round wire since we assume div E = 0. The analysis also assumes that Jr = σ Er but I know that this is wrong close to DC since at DC we have Jr = 0 and Er ≠ 0 inside the wire. This last fact is consistent with the true CPBC which says Jr(r=a,θ) = (jω) n(θ). The Hall surface charge is also ignored. Also, if I try to activate my loss model, I know it is only valid for small loss since it uses the TL equations. So all these things are reasons that the Appendix D results might be wrong at ω = 0. Finally I have some possible reasons!

Is there some easy way to "repair" Appendix D so it becomes more accurate at low ω?

Appendix D only uses E fields in the opening sections. So the first summary is still OK in terms of the unknown Km and am.  The first deviation occurs at (D.2.24) where I first use Jr = σ Er. So this would be a place where some kind of repair might be installed. From Appendix N we have

Jz =  σ ( Ez + ωcτ Er) 			ωc ≡  (qB/m)

Jr =  σ (Er - ωcτ Ez) 			B = B

Jθ = σEθ  .						σ =  (nq2τ/m)				(N.7.8)

but this is only for DC!  For general low ω AC, I don't know the connection between the J's and the E's.

OK, so I don't know how to do the repair, but I do have a REASON why the results won't be right at low ω. This then would explain the failure of the reflection model at low ω to give the correct two cylinders limit.

I am now ready to attempt an explanation of why my theory does not work for low ω. Then the question will be:  at what ω does the theory "drop out" ? Well, one answer would be that it fails when Jr = σEr fails. Or maybe when Er becomes on the order of Er,Hall. We know that

Er,Hall(r,θ) = Es(r/a)

Er(r,m)  = (j/4) ηm I Rdc (aβd) gm

Maybe simplify this to say

Er,Hall(r,θ)  ~  Es(r/a)				Es  ≡ Er(a) =       volts/m

Er(r,θ) ~   (j/4) I Rdc (aβd) g0			go ≈ 2 (r/a)  = 2

so then we get at r = a,

<<     (j/4) I Rdc (aβd) 2			valid

Thus, the limit will be I dependent.

<<     Rdc (aβd)  = 1/(σπa2) (aβd)  =  βd/(σπa)

<<  βd/(σ)

If we use the no-loss version of βd then

<<  ω/(vdσ)

ω >>

2πf >>

f  >>

I will be amazed if I have the units right.

dimRHS = henry/m * amps * m/sec * 1/ohm * 1/m * 1/m2 * m3 * 1/ Cou

= henry * amps * 1/sec * 1/ohm * 1/Cou

=ohm-sec * amps * 1/sec * 1/ohm * 1/Cou

= amps * 1/Cou  = sec-1  yes!

Let's try some numbers for Belden 8281 cable

μ0 = 4π x 10-7 henry/m

I = 1 amp

vd = 2 e 8  m/sec

σ = 	5.81 x 107 mho/m

a = 394e-6 m   ?

n = 8.5 x 1028  electrons/m3

q =  1.6 x 10-19 Coul

Here is Maple on this:

The result is 350 KHz. Recall that fc for this same cable was about 7800 Hz. This is much higher than I wanted to see it be!

What does this give for power line with a = 1e-2 m  (1 cm) and I = 100A?  I get 54 KHz which is not at all what I wanted to see!

Question:  The loss model says βd = (ω-jωc)/vd compared to βd = ω/vd for the no loss model.  If I go back to the above Hall condition

<<  βd/(σ)

and if I use the loss thing on the RHS, then it says

<<  ωc/(vdσ)

and this then puts a low limit on current I guess for which App D works. But then I get the results in my examples that I need ωc >> 350 KHz for Belden.