Crazy idea on Jm and the homo solutions			PhL		1.15.14

Again, this general idea is now installed as Section B.6 (b)

Conjectured Theorem:  If you add Jm to your Jc, the extra Jm integral gives exactly the right homo solutions to make the Az boundary conditions match.

Attempted Proof:  Start with Appendix C.4's starting point

Az(x) =  ∫ dV' [μ2Jcz(x') + μ0Jmz (x')] 			R = |x - x'|   	ω = 0		(1.5.9)

= Az(c) + Az(m)

I will show later that one can add an arbitrary surface current and that only changes the homo solutions, and I choose the current shown above.

1. I know that the "conduction current term" has this property at the boundary between μ1 and μ2:

[Az(c)]1  = [Az(c)]2			1 = dielectric = outside wire

[∂n Az(c)]1  = [∂n Az(c)]2

Therefore I know that

[∂n Az(c)]1  –  [∂n Az(c)]2   =  [-][∂n Az(c)]

This is the quantity that we want the Jm term to cancel out.

2. The "magnetization current term" is this

Az(m) = μ0 ∫ dV' Jmz (x')]

where Jmz (x')  is confined to the conductor surface and this has the usual (-∞,∞) dz' integral. The mag current density is in the form of a 3D density of amps/m2.

However, with the usual set of assumptions I can replace this 3D integral and propagator with as 2D integral and propagator as follows:

Az(m) = μ0 ∫ dA' Jmz (x')] [ -  ln(R) ]

Now since we know that Jmz (x') is confined to the 2D perimeter in this 2D picture, we then write

Az(m) = μ0 ∫C ds' Kmz (x')] [ -  ln(R) ]

where we have now "taken out the delta function" so K is amps/m. I showed in Appendix B that

Kz = -  (  -   ) Hθ		where Hθ > 0 and Kz ≤ 0.

so we then have

Az(m) = - (μ2-μ1) ∫C ds' Hθ(x')] [ -  ln(R) ]

= ∫C ds' [(μ1-μ2) Hθ(x')] [ -  ln(R) ]

= ∫C ds' a(x')] E(x|x') .		a(x') = (μ1-μ2) Hθ(x')

3. Now recall from Stakgold these facts

u(x) = ∫σ dSξ a(ξ) E(x|ξ)

u(s) = ∫σ dSξ a(ξ) E(s|ξ)

∂νu(x) = ∫σ dSξ a(ξ) ∂νE(x|ξ)	∂ν  = normal to surface σ  = I write as ∂n

∂νu(s)  =  [∂νu(x)]x=s+ = ∫σ dSξ a(ξ) ∂νE(x|ξ)  - a(s)/2		// the extra term!

where σ is a surface of interest. We take now this surface to be our perimeter and then E is the 2D E. Thus we expect to find that

[∂nAz(m)(x)]1 = ∫C ds' [(μ1-μ2) Hθ(x')] ∂nE(x|x') - [(μ1-μ2) Hθ(x)]/2   (**)

[∂nAz(m)(x)]2 = ∫C ds' [(μ1-μ2) Hθ(x')] ∂nE(x|x') + [(μ1-μ2) Hθ(x)]/2

Now compute our "correction term" for the difference boundary condition,

[∂nAz(m)(x)]1  –   [∂nAz(m)(x)]2

= {∫C ds' [(μ1-μ2) Hθ(x')] ∂nE(x|x') [- ]   - [(μ1-μ2) Hθ(x)] [+ ]/2

= (μ1-μ2) [- ] ∫C ds' Hθ(x')] ∂nE(x|x')  - (1/2) (μ1-μ2) [+ ] Hθ(x)

This then is our candidate correction term which we want to cancel the original term. I am not sure how that is going to happen!  The original was this:

[∂n Az(c)]1  –  [∂n Az(c)]2   =  [- ][∂n Az(c)]

Thus this is what I WANT to be true (add above two equations and hope for 0)

(μ1-μ2) [- ] ∫C ds' Hθ(x')] ∂nE(x|x')  - (1/2) (μ1-μ2) [+ ] Hθ(x) + [- ][∂nAz(c)]= 0

The μ factors don't look right, but let's plough ahead . Write

[- ]   = (μ2- μ1)/(μ1μ2)  = - (μ1-μ2) /(μ1μ2)

on the far right only, then cancel one factor  (μ1-μ2) to get

[- ] ∫C ds' Hθ(x')] ∂nE(x|x')  - (1/2) [+ ] Hθ(x) - [∂n Az(c)] =  0

Further simplify to

(μ2- μ1) ∫C ds' Hθ(x')] ∂nE(x|x')  - (1/2) (μ2+μ1) Hθ(x) - [∂n Az(c)] =  0		(*)   ??

I have obtained the above equation in my Section B.6 writeup.

Once again, I am trying to show that (*) is true, I have not shown it is true! I do know from (**) that

∫C ds' [(μ1-μ2) Hθ(x')] ∂nE(x|x')  = [∂nAz(m)(x)]1  +   [(μ1-μ2) Hθ(x)]/2    agree

so then here is what I want to show is true:

-[∂nAz(m)(x)]1  -   [(μ1-μ2) Hθ(x)]/2 - (1/2) (μ2+μ1) Hθ(x) - [∂n Az(c)] =  0		??

or

[∂nAz(m)(x)]1  +   [(μ1-μ2) Hθ(x)]/2 + (1/2) (μ2+μ1) Hθ(x) + [∂n Az(c)] =  0		??

or

[∂nAz(m)(x)]1  +  μ1 Hθ(x) +  [∂n Az(c)] =  0		??

Now how is Hθ related to Az ?  If we define a little local cylindrical coordinate system, then we know that

B = curl A =  [ r-1∂θAz - ∂zAθ] +  [∂zAr - ∂rAz] +  [ r-1∂r(rAθ) - r-1∂θAr

=  [ r-1∂θAz] +  [- ∂rAz]

For a circular boundary this would be just B =  [- ∂rAz] since Az is constant on the surface. Let us now conjecture that for a general perimeter we have

B =  [- ∂nAz ]

where   is a local tangent vector as I often use. Then we could claim that

Bθ = - ∂nAz  .

What Az is this?  It should be the "true one" where BC's are met, so this says

Bθ = - ∂nAz(c) - ∂nAz(m)

But is this inside or outside? I guess we have to say

Bθ1 = - [∂nAz]1

Bθ2 = - [∂nAz]2

Then we can write

H(θ) = Bθ1/μ1  = Bθ2/μ2  		take your choice

So I choose 1 and outside to get

H(θ) = Bθ1/μ1  =  -(1/μ1) [∂nAz(c) + [∂nAz(m)]1]

or

μ1 H(θ) = Bθ1  =  - [∂nAz(c) + [∂nAz(m)]1]

Now try this in our ?? equation:

[∂nAz(m)(x)]1  +  μ1 Hθ(x) +  [∂n Az(c)] =  0		??

[∂nAz(m)(x)]1  -    [∂nAz(c) + [∂nAz(m)]1] +  [∂n Az(c)] =  0		??

0 = 0 			!!!!!

Did I just go in a circle, or have I really succeeded here? Time is 8:13 AM 1.16.14.