Problems in Appendix B					PhL		10.14.14

I have found some new forms of Maxwell's implied equations.

1. Allocation of E and J in the curl H equation

Consider the Maxwell equation normally written as		ξ ≡ ε - jσ/ω  = ε + σ/jω

curl H =  ∂tD + J = jωD + J   = jωεE + J

Using Ohm's Law, we can write the RHS three different ways

1 	 jωεE + J

2 	jωε(J/σ) + J  = [  jω(ε/σ) + 1] J  = ( jω/σ)  [ ε + σ/jω ]   = jω(ξ/σ) J

3	 jω(ξ/σ)σE  = jωξ E

So here are our three ways to write the curl H equation:

1	curl H =    jωεE + J

2	curl H =    jω(ξ/σ) J   =

3	curl H =   jωξ E

I have checked with Maple to make sure the three RHS's are the same.

2. Forms of the Helmholtz equation

Consider this identity,

curl curl H  = grad divH -2H  = -2H

which then says

2H  = - curl curl H

This equation can then be written three ways using the three forms shown above. I will do them one at a time:

1	2H = - curl [jωεE + J]  = -jωε curl E - curl J  =  -jωε [-jωB ] - curl J  = -ω2εμH - curl J

so that we end up with		(2 + ω2εμ)H = -curl J

or					(2 + k2 )H = -curl J 		with	 k2 = ω2εμ

This form appears in lines doc here:

(2+k32)E = μ3jωJ3	(2+k32)B = - μ3 curl J3	// region 3   ki2 = ω2μiεi     	(1.5.26)

2 	2H = - curl [jω(ξ/σ) J] = 	- jω(ξ/σ) curl J		ξ ≡ ε - jσ/ω  = ε + σ/jω

This form does not appear anywhere in lines doc as far as I know. The ratio ξ/σ never appears.

3	2H = - curl[jωξ E]  = -jωξ curl E  =  -jωξ [-jωB]  = -ω2ξ B   = -ω2ξμ H

so that we end up with		(2  + ω2ξμ )H  = 0

(2  + β2 )H  = 0

This form appears in lines doc here.

(2+β32)E  =  0		(2+β32)B  = 0			// region 3   βi2 = ω2μiξi 	(1.5.27)

Here then are the three different "Helmholtz equations" which are the same:

1	(2 + k2)H = -curl J 		with	 k2 = ω2εμ

2	2H =  - jω(ξ/σ) curl J		ξ ≡  ε + σ/jω      jω(ξ/σ)  = jω([ε + σ/jω]/σ)  = jω(ε/σ) + 1

= - [1 + jω(ε/σ) ] curl J

3	(2  + β2 )H  = 0			β2 = ω2ξμ

3. Copper conductor case

Question:  For copper, what is the size of jω(ε/σ) compared to 1 ?  That is, when is ωε << σ  ? I answer this question below (2.2.2) and conclude that ωε << σ  as long as f << 1018 Hz.

Therefore, in Form 2 above, we really can neglect the jω(ε/σ) relative to the 1. Now go back to

curl H =  ∂tD + J = jωD + J   = jωεE + J  = [  jω(ε/σ) + 1] J

The fact that ωε << σ  in this range is the same as saying you can neglect the displacement current in copper relative to the conduction current.  This says

ξ ≡  ε + σ/jω  ≈  σ/jω		all the way up to 1018  Hz

Then

jω(ξ/σ)  = jω(1/σ) * σ/jω   = 1		β2 = ω2ξμ  = ω2(σ/jω)μ = -jμσω as usual

Then the three forms shown above become, in the limit ωε << σ,

1	( 2 + k2 )H = -curl J 		with	 k2 = ω2εμ

2	2H = - curl J

3	( 2  + β2 )H  = 0			β2 =  -jμσω

How do I explain how these can all be true at the same time?  Are they if ωε << σ ?

First consider

β2 =  -jμσω		k2  = ω2εμ		| k2/β2| = ωε/σ  << 1

so we do know that k << β in magnitude.

Comparing 1 and 2, I would like to show that

| k2 H | << | 2 H|

Using 3 this is the same as

| k2 H | << | β2 H|

and this is at once true since k << β . So that explains how 1 and 2 can both be true.

4. NOW take a look at Appendix B.2 (a)

I have added the above data to the start of Section B.2. I can then take two different paths. If I assume the e-jkz ansatz, then ∂z2 = -k2 and

2H = -curl J			// copper for f << 1018 Hz

becomes

(22D - k2) H  =  -curl J

but I don't know how to invert this equation very well. But I do know how to invert -2H = curl J  from Appendix H.1.8

-2 f(x) = s(x)		=>	f(x) = ∫d3x' [1/4πR] s(x')   + homogeneous solutions

The Poisson Equation									(H.1.8)

so then

H(x) = ∫d3x' [1/4πR] curl' J(x')

I think this result is good for all ω, whereas the result I wrote

H(x,y)  = ∫d2x' [ ln(1/R) ] curl' J(x')	R = |x-x'|					(B.2.5)

makes the assumption that J = Jz(x,y)  so e-jkz  ≈  1 which means low ω. So maybe I will make the replacement above. Let's continue and see what comes next in Appendix B before doing this.

5. Take a look at Appendix B.2 (b)

Here the starting equation is

(2 + β2)A(x) =   - μ J(x)  .			all of region R				(1.5.4)

STOP!  Who says the above equation is true inside the conductor??  I think the correct equation to be using here is this

(2 - με ∂t2)A  = - μJ		[ = Jackson (6.16) ]					(1.3.5)

which uses the Lorenz gauge.  I don't think the King gauge makes sense for a single conductor? Well, I show that with the large σ assumption, I do get this result

(2 - μ1ε1 ∂t2 - μ1σ1∂t  ) A  = - μ3J3  .			region 3			(1.3.21)

and I guess I can consider region 3 as my conductor. But in the King gauge, this yields,

(2 + βd2)A(x) =   - μ J(x)			!!

Even though we are inside the conductor, βd is what shows! Then it all works!

I could do a full 3D solution to this using

- (2+k2) f(x) = s(x)	=>	f(x) = ∫d3x' [e-jβR/4πR] s(x')  + homogeneous solutions

The Helmholtz Equation									(H.1.9)

so the solution would be

A(x) = μ ∫d3x' [e-jβR/4πR] J(x')

Now consider

B = curl A   =  μ ∫d3x' curl { [e-jβR/4πR] J(x') }

But we know that for Q a constant vector,

x (φQ) = φ x Q + φ ( x Q)   = φ x Q   = grad φ x Q

Thus

B = curl A   =  μ ∫d3x' grad [e-jβR/4πR]  x J(x')

so

H = ∫d3x' grad [e-jβR/4πR]  x J(x')

Hi = ∫d3x' [ grad [e-jβR/4πR]  x J(x') ]i

= - εirs ∫d3x' [∂'r [e-jβR/4πR]  Js(x')

= + εirs ∫d3x' [e-jβR/4πR]  ∂'r Js(x')

= + ∫d3x' [e-jβR/4πR] εirs ∂'r Js(x')

= + ∫d3x' [e-jβR/4πR] [curl' J'(x') ]i

which then says

H =   + ∫d3x' [e-jβR/4πR] curl' J'(x')

Compare this to the other method which said

H(x) = ∫d3x' [1/4πR] curl' J(x')

How do I then argue that both these results are the same for a good conductor?