App D Actions 2						PhL		11.30.13

This material is all in Appendix D now. I did a few checks to show new App D agrees with B calculations that were done here.

1. Our first task is to compute the B field	1

2. Our next task is to check the four Maxwell equations	2

3. Here are the B fields read off from the Maple computation	3

I showed in App D Actions 1 that when I computed the corresponding B field, I obtained a pair of fields E and B which satisfies all four of Maxwell's equations, but that was only for m = 0.

Here I would like to repeat that same process but for general m. I want to do this without assuming any boundary conditions. My E fields from App D are stated before any BC's are applied as"

jEφ(r,m)  = - am x-1 Jm(x) + ( +  ) Jm+1(x)         x = β'r	  .				(D.2.15)

Er(r,m)  = am x-1 Jm(x) +  Jm+1(x)  .							(D.2.11)

Ez(r,m) = Czm Jm(β'r)									(D.1.27)

Km ≡ 2j (βd/β') Czm										(D.2.5)

From the last we get

Czm = (1/2j)(β'/βd)Km  = -j (β'/βd)

Thus without any assertion of boundary conditions, our E field App D solutions for general m > 0 are

Ez(r,m) =  -j (β'/βd) Jm(x)			x = β'r

Er(r,m)  = am x-1 Jm(x) +  Jm+1(x)

jEφ(r,m)  = - am x-1 Jm(x) + ( +  ) Jm+1(x)

## 1. Our first task is to compute the B field

Now you have to remember that there is an outside factor of

ej(ωt-βz) ejmφ

so now we have two rules

∂z → -jβd

∂θ → +jm

We identify θ and φ as the same variable.

B = (-1/jω) curl E

curl E =  [ r-1∂θEz - ∂zEθ] +  [∂zEr - ∂rEz] +  [ r-1∂r(rEθ) - r-1∂θEr ]

=   [ r-1jmEz+jβdEθ] +  [-jβdEr - ∂rEz] +  [ r-1∂r(rEθ) - r-1jmEr ]

Maple has computed these three components, I won't copy them here right now but they have this form

B = Br(r)   + Bθ(r)   + Bz(r)

## 2. Our next task is to check the four Maxwell equations

(1) div B = 0.

div B   = r-1∂r(rBr) + r-1∂θBθ + ∂zBz

= r-1∂r(rBr) +r-1jmBθ -jβd Bz

Maple confirms that div B = 0, I get to use my unapply operator, seems good.

(2) div E = 0.

div E   = r-1∂r(rEr) + r-1∂θEθ + ∂zEz

= r-1∂r(rEr) + r-1jmEθ - jβdEz

Again Maple confirms. We just have verification within each partial wave since it could be activated all by itself.

(3) curl E  = -jωB, but this is how we produced B in the first place so OK.

(4) curl B = μ(σ+jωε) E

This one is the Big Enchilada. I should at the start make this replacement

μ(σ+jωε) = (-1/jω) β2

so what I really need to show is this:  				β'2+ βd2 = β2

curl B = (-1/jω) β2 E  =  (-1/jω)( β'2+ βd2) E

We start then with

curl B =  [ r-1∂θBz - ∂zBθ] +  [∂zBr - ∂rBz] +  [ r-1∂r(rBθ) - r-1∂θBr ]

=    [ r-1jmBz +jβdBθ] +  [-jβdBr - ∂rBz] +  [ r-1∂r(rBθ) - r-1jmBr ]

We want to show that

curl B + (1/jω) β2 E   = 0

or

[curlB]z + (1/jω) β2 Ez   = 0

[curlB]r + (1/jω) β2 Er   = 0

[curlB]th + (1/jω) β2 Eth   = 0

Maple confirms all three of these equations!  This is a major happy thing for me right now.

## 3. Here are the B fields read off from the Maple computation

Bz =  (β'/ω) ( +  )Jm(β'r)

Br = (j/ω)( r-1)[ (β'/βd) m - (βd/β')am ] Jm(β'r)    +    (j/ω)βd (+ ) Jm+1(β'r)

Bθ = - (1/ω)( r-1)[ (β'/βd) m - (βd/β')am ] Jm(β'r) +    (1/ω)  (βd + β'2/βd) Jm+1(β'r)

Rewrite to simplify if possible		// these three match box (D.4.9) of new App D

Bz =  (β'/ω) ( +  )Jm(β'r)

Br = (j/ω)( r-1)[ (β'/βd) m - (βd/β')am ] Jm(β'r)    +    (j/ω)βd (+ ) Jm+1(β'r)

Bθ = - (1/ω)( r-1)[ (β'/βd) m - (βd/β')am ] Jm(β'r) +    (1/ω)β'  (βd/β' + β'/βd) Jm+1(β'r)

or

Bz =   (1/ω)β' ( +  )Jm(x)

Br =   j (1/ω) βd[ (β'2/βd2) m - am ] Jm(x)/x    +    j (1/ω)βd (+ ) Jm+1(x)

Bθ = -  (1/ω) βd[ (β'2/βd2) m - am ] Jm(x)/x     +      (1/ω) βd  (1 + β'2/βd2) Jm+1(x)

I don't know how large the constants are, so not sure what terms are large.

For m = 0 these say			// these match (D.6.1)

Bz =   (1/ω) β' ( )J0(x)

Br =  j (1/ω)βd () J1(x)

Bθ =  (1/ω) βd  (1 + β'2/βd2) J1(x)

For β >> βd we can write these as

Bz =     (1/ω) βd (β'/βd) ( )Jm(x)		medium

Br =    j (1/ω)βd () Jm+1(x)		small

Bθ ≈     (1/ω) βd (β'2/βd2)  J1(x)		large

I know from actions 1 v2 that

K0 = jI βd/(πσa) * 1/ J1(β'a)    = jI (aβd)/(πσa2) * 1/ J1(β'a)  = j (aβd) I Rdc/ J1(β'a)

So my m=0 B field components become

Bz =     (1/2ω) β' (K0 )J0(x)

Br =    j (1/2ω)βd (K0) J1(x)

Bθ =     (1/2ω) βd K0 (1 + β'2/βd2) J1(x)

or

Bz =     j (1/2ω) β'  (aβd)  I RdcJ0(x) / J1(β'a)

Br =    -  (1/2ω) βd (aβd)  I Rdc J1(x) / J1(β'a)

Bθ =     j (1/2ω) βd  (aβd) I Rdc (1 + β'2/βd2) J1(x) / J1(β'a)

Again in the β >> βd limit these become

Bz =     j (1/2ω) β'          (aβd)  I Rdc J0(x) / J1(xa)		medium

Br =    -  (1/2ω) βd         (aβd)  I Rdc J1(x) / J1(xa)		small

Bθ =     j (1/2ω) β'(β'/βd) (aβd)  I Rdc (J1(x) / J1(xa)		large

Now I can rewrite the last as

Bθ ≈     j (1/2ω) β2 a  I (1/πσa2) (J1(x) / J1(xa)  = 	j (1/2ω) [ω2μσ/jω] I (1/πσa) (J1(x) / J1(xa)

=    ([μ] I (1/2πa) (J1(x) / J1(xa)

and this is the Chapter 2 result.

Here then is my general-m summary of all fields

Ez(r,m) =  -j (β'/βd) Jm(x)			x = β'r

Er(r,m)  =    am Jm(x)/x +  Jm+1(x)

jEφ(r,m)  = - am Jm(x)/x + ( +  ) Jm+1(x)

(ω/β')Bz(r,m)   =   ( +  )Jm(x)

(ω/jβd)Br(r,m) =    [ (β'2/βd2) m - am ] Jm(x)/x   +  ( + ) Jm+1(x)

(ω/βd)Bθ(r,m)  = -  [ (β'2/βd2)  m - am ] Jm(x)/x    +   (1 + β'2/βd2) Jm+1(x)

And they satisfy the four Maxwell equations exactly.  I don't know how to better state these fields.