What about the Helmholtz Eφ equation?

This stuff is now incorporated  as Appendix section D.3.

Here are the Er and Eφ equations in m space:

[2E]r + β2 Er = 0 :

[r2∂r2 + r∂r - (m2+1) + r2(β2-βd2)] Er(r,m) - 2jm Eφ(r,m) = 0				(D.1.17)

[2E]φ + β2 Eφ = 0 :

[r2∂r2 + r∂r - (m2+1) + r2(β2-βd2)] Eφ(r,m)  + 2jmEr(r,m) = 0				(D.1.18)

How do I know that my solutions Eφ and Er satisfy this last equation? Maybe they don't! Why should they?

Rewrite the above in this manner

[x2∂x2 + x∂x - (m2+1) +x2] Er(r,m)  - 2m jEφ(r,m) = 0				(D.1.17)

[x2∂x2 + x∂x - (m2+1) + x2] jEφ(r,m)  - 2mEr(r,m) = 0				(D.1.18)

I solved the first equation for Eφ in my work, and I solved the Hz equation for Er. So what happens when I insert these two solutions into the last equation above?  Rewrite the last as

[x2∂x2 + x∂x - (m2+1) + x2] jEφ(r,m) = 2m Er(r,m).

It would certainly be nice if this were true. I have

Er(r,m)  = am x-1 Jm(x) +  Jm+1(x)  .						(D.2.11)

jEφ(r,m)  = - am x-1 Jm(x) + ( +  ) Jm+1(x)         x = β'r	  .				(D.2.15)

Then we get

LHS = [x2∂x2 + x∂x - (m2+1) + x2] {- am x-1 Jm(x) + ( +  ) Jm+1(x) }

RHS = 2m{ am x-1 Jm(x) +  Jm+1(x)}

You would think this has to balance separately as if Km and am were independent constants. So then

LHSK = [x2∂x2 + x∂x - (m2+1) + x2] (  Jm+1(x) )

RHSK = 2m  Jm+1(x)

Then I would have to show that

[x2∂x2 + x∂x - (m2+1) + x2] Jm+1(x) = 2m Jm+1(x)

or

[x2∂x2 + x∂x - (m2+1) + x2 - 2m] Jm+1(x) = 0

or

[x2∂x2 + x∂x - (m2+1+2m) + x2] Jm+1(x) = 0

or

[x2∂x2 + x∂x - (m+1)2 + x2] Jm+1(x) = 0

or

[x2∂x2 + x∂x  + x2- (m+1)2] Jm+1(x) = 0

Bessel's equation does say that

[x2∂x2 + x∂x  + x2 - (m+1)2 ] Jm+1(x) = 0

so it works!!!!!  I am totally amazed. At least the Km terms balance.

Now let's try the am balance.

LHS = [x2∂x2 + x∂x - (m2+1) + x2] {- am x-1 Jm(x) +   Jm+1(x) }

RHS = 2m{ am x-1 Jm(x) }

LHS = [x2∂x2 + x∂x - (m2+1) + x2] {- x-1 Jm(x) +  m-1 Jm+1(x) }

RHS = 2m{ x-1 Jm(x) }

This is a much messier situation. I could try Maple. Here is Maple evaluating the LHS:

But this is exactly the RHS !!  I am again astounded to see this working right.

Now I can consider the notion of trying to find the m = 0 solution for Eφ using the φ equation,

[r2∂r2 + r∂r - (m2+1) + r2β'2] Eφ(r,m)  + 2jmEr(r,m) = 0

which for m = 0 becomes

[r2∂r2 + r∂r -1 + r2β'2] Eφ(r,m=0)  = 0

or

[x2∂x2 + x∂x -1 + x2] Eφ(r,m=0)  = 0

This seems to say that

Eφ(r,0)  = Cφ0 J1(x)

where Cφ0 is some constant.

How does this then figure into my boundary conditions stuff? If I want this to be zero on the surface using my equipotential idea, then for m =0 I must conclude that Cφ0 = 0. But that is not required, and I suppose you could allow an m = 0 rotating current on the surface?

Question:  Can the φ Helm be derived from the others and div E = 0?

[2E]z + β2 Ez = 0 :

[r2∂r2 + r ∂r - m2 + r2 β'2] Ez(r,m)   =  0		  ok				(D.1.16)

[2E]r + β2 Er = 0 :

[r2∂r2 + r∂r - (m2+1) + r2 β'2] Er(r,m) - 2jm Eφ(r,m) = 0				(D.1.17)

[2E]φ + β2 Eφ = 0 :

[r2∂r2 + r∂r - (m2+1) + r2β'2] Eφ(r,m)  + 2jmEr(r,m) = 0				(D.1.18)

div E = 0 :

∂r [r Er(r,m)] + jmEφ(r,m) -jβd r Ez(r,m) = 0							(D.1.19)

Maybe ponder in Cartesians first.

(∂j2 + β2) Ei= 0

∂jEj = 0

(∂12+∂22+ ∂32 + β2) E1= 0

(∂12+∂22+ ∂32 + β2) E2= 0

(∂12+∂22+ ∂32 + β2) E3=  ????

Start with

∂1E1 + ∂2E2+ ∂3E3 = 0

then

(∂12+∂22+ ∂32 + β2)∂1E1 + (∂12+∂22+ ∂32 + β2)∂2E2+ (∂12+∂22+ ∂32 + β2)∂3E3 = 0

or

∂1 (∂12+∂22+ ∂32 + β2) E1 + ∂2 (∂12+∂22+ ∂32 + β2) E2+ ∂3 (∂12+∂22+ ∂32 + β2) E3 = 0

or

∂1 0 + ∂2 0+ ∂3 (∂12+∂22+ ∂32 + β2) E3 = 0

or

∂3 [(∂12+∂22+ ∂32 + β2) E3] = 0

Well it certainly works but not a hard proof. Go back to

(2+ β2)E = 0	=>	grad(div E) – curl (curl E) + β2 E = 0

When div E = 0 this says

– curl (curl E) + β2 E = 0