E fields outside the round wire?				PhL		10.5.13

This has now been installed in appendix D, no need to keep long term.

How is Appendix D different if you are thinking outside the wire? Just page through the appendix.

First of all, the β appearing in the Helmholtz is then βd  since we are "in the dielectric". So we have

( 2 + βd2 ) E(r,φ z, t) = 0 .									(D.1.1)

Assume the same form

E(r,φz,t) = ej(ωt-βz) E(r,φ) .								(D.1.2)

As I read through the Helmholtz section, what comes out at the far end is this:

Summary of Examples:

[2E]z + β2 Ez = 0 :

[r2∂r2 + r ∂r - m2] Ez(r,m)   =  0								(D.1.16)

[2E]r + β2 Er = 0 :

[r2∂r2 + r∂r - (m2+1)] Er(r,m) - 2jm Eφ(r,m) = 0						(D.1.17)

[2E]φ + β2 Eφ = 0 :

[r2∂r2 + r∂r - (m2+1)] Eφ(r,m)  + 2jmEr(r,m) = 0					(D.1.18)

div E = 0 :

∂r [r Er(r,m)] + jmEφ(r,m) -jβd r Ez(r,m) = 0							(D.1.19)

So basically you set β' = 0 it seems to me.

Now down to the Ez solution.

[r2∂r2 + r ∂r - m2] Ez(r,m)   =  0

[∂r2 + (1/r) ∂r - m2/r2] Ez(r,m)   =  0

[r∂r2 +  ∂r - m2/r] Ez(r,m)   =  0

This is not Bessel's equation!  Ouch, what is it?

r∂r(r∂rf(r))  = m2f(r)

How do I "look up" an ODE. Polyanin is so massive. Here is an offering he has

[r2∂r2 + r ∂r - m2] Ez(r,m)   =  0		a = 1 and b = -m2

Then μ = 1/2*| 0 - 4(-m2)|1/2 = (1/2) | 4m2| = (1/2)2m = m. The if condition is 0 > 4b = -4m2 so

Ez(r,m) = C1|r|m + C2|r|-m  = Arm + B r-m

Yes, this is the radial equation for the 2D Laplace equation 2u = 0 as on Stak II page 90, equation (7.8) page 92. I never knew it by the name Euler Equation. For m = 0 it is the second case and the solutions are A + Bln r as I now well remember.