Electric Field Outside a Round Wire at DC				PhL	12.11.13

My conclusion is shown in the picture below, I think this issue is resolved. I found a physical situation which gives fields which satisfy the Max equations. The Jackson paper reference is noted.

Question 1.  Current I flows in a DC round wire of radius a.

(a) What is the Ez field just outside the surface?

(b) What is Ez outside the wire?

(a) I think the current profile is uniform and just drops to zero as a step function at r = a. If so, then we have Ez = J/σ  at r = a-ε  where J = I/(πa2). If Ez is continuous across the boundary as I think it is, then the answer is that just outside the wire, Ez = I(πa2σ).  [ I still think this is correct ]

(b) There are perhaps several approaches here. We seem to know that dive E = ∂zEz(r) = 0 if there are no other field components in existence, but that fact just says Ez does not vary with z which we already know. We know that

(2 - με ∂t2)E  =  μ∂tJ + (1/ε) grad ρ							(1.2.1)

In our case, outside the wire, I assume J = 0 since perfect insulator say, and ρ = 0 and E is static, so

2Ez(r) = 0

or

22DEz(r) = 0

This means that we must have

Ez(r) = A + B ln (r/a)		ok

Then the boundary condition says

Ez(a)  = A = I(πa2σ)			ok

So then we have

Ez(r) = I(πa2σ) + B ln (r/a)

What sets the other constant?  Maybe one of the Maxwell equations?

curl E =  0

curl E =  [ r-1∂θEz - ∂zEθ] +  [∂zEr - ∂rEz] +  [ r-1∂r(rEθ) - r-1∂θEr ]

= +  [- ∂rEz]

This seems to say that ∂rEz(r)= 0 everywhere and that would set B = 0.  ok

Well, we know there is an H field outside the wire given by

2πrHθ = I		Hθ(r) = I/2πr.

We should have curl H = 0 since ∂t = 0 and J = 0 outside, so then

curl H =  [ r-1∂θHz - ∂zHθ] +  [∂zHr - ∂rHz] +  [ r-1∂r(rHθ) - r-1∂θHr ]  = 0

+  [ r-1∂r(rHθ) ]  = 0		// and this works properly since Hθ(r) = I/2πr.

So my tentative conclusion is rather astounding. It says Ez(r) = I(πa2σ) everywhere outside the wire, all the way out to r = ∞.

Conclusion:  I think this is correct, that Ez = constant in all 3D space for this problem. Here is my interpretation of this solution:  This uniform Ez field is provided by a huge parallel plate capacitor at the ends of the round wire (at ±∞). The capacitor makes Ez uniform everywhere between the plates. It just happens that in the round wire it causes a current to flow.  This is certainly A possible Maxwell equation solution, and I "found it" above but in retrospect it is quite obvious.

"Surface charges on circuit wires and resistors play three roles" by J. D. Jackson, in American Journal of Physics -- July 1996 -- Volume 64, Issue 7, pp. 855 .

Scribd has it but I am too tired to upload something.

There are lots of fancy papers on this subject addressing induced charge, relativistic effects and so on. But for me, I think the answer is this:

For an idealized isolated infinite round wire, Ez = constant everywhere in space. This is consistent with Maxwell's Equations. Notice above how the curl E equation killed off the lnr term.

In the more realistic situation of two oppositely directed wires, if they were forced to have a uniform current density even, things are messier. Now we have surface charges and external fields etc.

The notion of fields outside a uniform wire is slippery I feel. 		agreed

Question 2.  Can a round wire have an Eφ ≠ 0 field at the surface?

I keep coming back to this painful question. If I argue that Eφ = 0 due to "quasi-static", then why doesn't that mean Ez = 0 as well at the surface. Hold on this question.  [ Section D.8 at least faces this question ]

Question 3.  In the round wire with wave going down it, there is Ez at the surface I think. I know that Ez creates a current Jz inside the wire and pushes those conduction electrons along. Does this Ez push on the surface charge density and make it do something?  Does it make a surface current in the z direction? If this surface charge is really in a little channel at the surface, then yes, there should be a surface current. The free electrons then oscillate back and forth in the z direction along with z.  [ Final Reader Exercise ]