What is A and φ for the Round Wire of Chapter 2 ?		PhL		10.19.13

Comments 12/5/13. This seems to be the first time I ever tried this calculation, notice early date. Even now this subject is up in the air. I have something in Appendix B, but have not "gotten to it" yet. Remember that Chapter 2 is E and B field oriented, there is no potential. There ought to be a way to get that same result using potential theory, you would think. I later tried this in my "illogical" uniform J Helmholtz treatment. The King method requires a prescribed current, and that just doesn't work well with this problem where one of the "outputs" of the calculation is the current J and its skin effect.

This is something I never computed because that section was done using fields only. Remember that we always have gauge freedom for the potentials.

In Chapter 2 I assume that

E = E = E(r)

B =B  = B(r)

Now let's look at our defining relations:

B = curl A		E = - grad φ - ∂tA  .							(1.3.1)

We than have

B(r) = [curl A]  	 = ∂zAr - ∂rAz

E(r) = -∂zφ(r) - ∂tAz(r)  = -∂zφ(r) -jω Az(r)

We are allowed to pick any gauge we want, so perhaps

div A = 0 = ∂zAz + (1/r)∂r(rAr) + (1/r)∂θAθ

In addition to the above, we also have

[curl A]  	  = 0		=>	(1/r) ∂θAz - ∂zAθ  = 0

[curl A]  	  = 0		=>	(1/r)∂r(rAθ) - (1/r)∂θAr = 0

[- grad φ - ∂tA]  	  = 0	=>	- ∂rφ - ∂tAr = 0	=>	- ∂rφ -jωAr = 0

[- grad φ - ∂tA]  	  = 0	=>	(1/r)∂θφ - ∂tAθ = 0 	=>	(1/r)∂θφ - jωAθ = 0

So here is a summary of all the equations in the ω domain

(1/r) ∂θAz  =  ∂zAθ						1

∂r(rAθ)  =  ∂θAr						2

∂rφ = -jωAr						3

∂θφ = rjωAθ						4

∂zAr - ∂rAz = B(r)						5

∂zφ(r) +jω Az(r) = - E(r)					6

∂zAz + (1/r)∂r(rAr) + (1/r)∂θAθ  = any function you want	7

One starting point is to require that ∂θφ = 0 as a symmetry condition, which from 4 says Aθ= 0. The equations then simplify to this smaller set

(1/r) ∂θAz  =  0						1

∂θAr  = 0							2

∂rφ = -jωAr						3

∂θφ = 0							4

∂zAr - ∂rAz = B(r)						5

∂zφ +jω Az = - E(r)						6

∂zAz + (1/r)∂r(rAr)  = any function you want		7

Now 1 says that Az = Az(r,z).

Now 2 says that Ar = Ar(r,z)   and this is true for all functions.

Question:  In item 7, why can't I select "any function" to be (1/r)∂r(rAr). Then the gauge condition says

∂zAz= 0 		=>	Az = Az(r) only

The equations of interest are then

∂rφ(r,z) = -jωAr(r,z)					3

∂zAr(r,z) - ∂rAz(r) = B(r)					5

∂zφ(r,z) +jω Az(r) = - E(r)					6

Suppose I use 3 to eliminate Ar. then

∂z[-∂rφ(r,z)/jω] - ∂rAz(r) = B(r)				5

∂zφ(r,z) +jω Az(r) = - E(r)					6

or

(1/jω)[∂z∂rφ(r,z)] - ∂rAz(r) = B(r)				5

∂zφ(r,z) +jω Az(r) = - E(r)					6

Now use 6 to eliminate ∂zφ to get

(1/jω)[∂r{ - E(r)  - jω Az(r)}] - ∂rAz(r) = B(r)		5

or

(1/jω)[∂r{ E(r)  + jω Az(r)}] + ∂rAz(r) =  - B(r)

or

[∂r{ E(r)  + jω Az(r)}] + jω∂rAz(r) =  - jωB(r)

or

∂rE(r)  + jω ∂rAz(r) + jω∂rAz(r) =  - jωB(r)

or

∂rE(r)  + 2jω ∂rAz(r) =  - jωB(r)

or

2jω ∂rAz(r) =  - jωB(r) - ∂rE(r)  ≡  F(r)

But in my round wire solution (2.1.9) I found that

∂rE(r)  = jωB(r) .										(2.1.9)

so therefore

2jω ∂rAz(r) =  -  ∂rE(r) - ∂rE(r)  ≡  F(r)  = -2 ∂rE(r)

This tells me that

jω ∂rAz(r) = -∂rE(r)

jωAz(r) = - E(r) + constant

Now look again at

B = curl A		E = - grad φ - ∂tA  .						(1.3.1)

You can always shift A by a constant and not affect the E and B fields. This is in addition to the gauge choice you might make which involves only derivatives of A. So write

jωAz(0) = - E(0) + constant  = 0

So we set the constant to E(0) = E(a)/ J0(βa). Then we have

jωAz(r) = - E(r) + E(0)

Az(0) = 0.

So there is my Big Solution for Az(r).

Az(r) = -(1/jω) [E(r) - E(0) ]  = (j/ω) [E(r) - E(0)]

STOP. I want to see someone else do this problem!