Dealing with Homo Solutions in a Transmission Line		PhL		11.24.13

Comments 12/5/13. This doc shows how I was struggling with the issues related to the homo solutions question and why King seems to do all his work never adding homo solutions and just using the Helmholtz integral for Az. I first wondered if it was the fact that total current is 0 in a full transmission line and maybe that makes the homo terms go away (no, that is not it). I now think it is the fact that in his analysis, there is only one μ for dielectric and conductors, and in this case, the Az and ∂nAz boundary conditions are "automatically met" by the Helmholtz integral, so there is no need to add homo solutions. This is not the case when μc ≠ μd, and this is a case King never treats.

I want to be able to ignore homo solutions somehow so I can use the Helmholtz integral by itself to compute parameters of a transmission line.

Background:  In my off-line analysis of a round wire with a uniform current  (which is the low frequency limit of an actual wire with AC), I found that the particular integral does NOT give the entire solution. You have to add a homo solution A ln(r) outside, and a homo solution B inside, and then match boundary conditions to determine A and B.

## Plan A. Is it somehow the sum of the two conductors that makes the homo terms go away?

Now here is one argument that might be a hint. Consider a transmission line with two parallel conductors not one inside the other. They are not round.  Suppose in the dielectric region there is some homo solution required to match the BC's. Write that as  (for φ or Az)

φhomo(x)  = A F(x)

where F(x) is some complicated function perhaps for a weird shaped wire cross section. As we move very far away laterally from the transmission line, the two conductors "look like" round wires going down the central axis of a cyl coord system. In this large r case, we know that, if the currents are balanced, there is no ln(r) term in the total solution. So we know that

limit(large r) of [ φpart + φhomo ]  ≠  proportional to ln(r)		in dielectric   [ ok ]

Now in this limit, I know that for a single round wire with uniform current, both φpart AND φhomo have ln(r) terms in them. I expect that to be true more generally for non-round wires. So I am claiming that when we include all conductors and include part + homo, there will be no ln(r) term. This does not say φhomo = 0. So you could have, for large r, some φhomo which cancels out φpart in terms of the ln(r) component.

What I do know is this:  in the distance region, if there IS a φhomo term present, it  can only have the general form A ln(r) + B in the large r limit. This is because in that limit, these are the only atoms for a solution of the 2D Laplace equation. Somehow I think I can argue that both A and B must be zero for a current balanced line. Let's assume that for the moment. Could probably show it for twin lead. So I guess my claim is this

(1) 		φhomo  → A lnr + B		for large r		only possibility

(2) 		A = 0 and B = 0

(3) 		φhomo = C F(x)    for non-large r, function F(x) complicated

Then I guess I am saying

limit large r [ C F(x)]  = 0

Either C = 0  or F(x) → 0.  Write an expansion for F(x) in some arbitrary cyl system whose center line is near the two conductors. In this expansion, we have an infinite series of terms with atoms ln r, 1, r, r2, r-1, r-2 and so on. I suppose we could have this general expansion.

limit large r [ F(x)]  = A lnr + B  + Cr-1 + Dr-2 ....

BUT, for large r, we have axial sym, which rules out things like C and D? This is a crucial question so be careful. I can see if two ways

(1) for large r, the terms Cr-1 + Dr-2  → 0 even if C and D are not zero. At any finite r, there is some small amount of axial asym, so these terms could be present. So you cannot argue that C and D are 0.

(2) you can argue that C and D are 0.

Coaxial line. Imagine that dielectric exists both inside and outside. Then at large r, the above argument still holds and we conclude that φhomo = 0.

So my conclusion is this:  For a total current = 0 transmission line of any kind, φhomo = 0. Now what about φpart ?  Since it is not a Laplace solution, the above argument does not work and it could be non-zero.  Well let's play the hand. Consider φpart. It is NOT a solution of Laplace, so I don't know that it has the Laplace

Discovery:  On page 11 King says "the solutions (23) and (24) take account of the boundary conditions automatically".  I read the following 2.5 pages and I see no attempt on his part of back up this claim! But I do hope it is true and that is really what I am trying to show here.  That is, I am trying to show that you don't need to add any homo terms. At least King is telling me that what I want to be true is in fact true.

[ somewhere later I show that this King claim is valid if μ1= μ2 since then "nothing really happens" at the boundary in terms of magnetic things like B and Az ]

I think I am one right track above but I need a way to decide that (2) is the right answer.

Analogy with a Line Charge in Electrostatics

Digression. Consider a line source of charge in 2D. Gauss's Law says 2πrEr = λ and so Er = λ/2πr and then φ = - (λ/2π) ln(r).  Far away from the line charge, we have this Laplace solution and this is an allowed atomic form, and there is no constant (the other allowed form).

Now suppose the line charge is really a thin wire of some weird cross section. We know that outside the wire we can for sure write the potential using this kind of expansion

φ = A lnr + B + Σm=1∞ (Amrm + Bmr-m) eimφ

since this is the most general Laplace solution. We know at once that Am = 0 since if Am were not zero, we would have a power blow up of the potential.  We know that in the limit of large r, we have φ = A lnr. Does this somehow tell us that B and Bm vanish???  So this is perhaps a simpler problem to think about.

I would say that for sure at least some Bm must NOT vanish, otherwise close in you would have an azisym solution which we know is not true for a square cross section wire! So we then have for our wire of non-round cross section

φ(r) = A lnr + B + Σm=1∞ Bm r-m eimφ

So for a single conductor, I think this is the potential outside in a 2D analysis with Bm ≠ 0.

If we consider then two conductors closely spaced but opposite current, and cyl axis is some average center line say. Then we would have

φ(r) = A lnr + B + Σm=1∞ Bm r-m eimφ

φ'(r) = -A' lnr - B' - Σm=1∞ B'm r-m eimφ

where the prime and nonprime coefficients are close in value. Then we have

φtot = φ + φ'  =  (A-A') lnr + (B-B') + Σm=1∞ (Bm-Bm') r-m eimφ

I just don't see how you can rule out these  (Bm-Bm') contributions! In fact, if we did an exact solution to a twin lead problem, I would guess we have some resulting coefficients.

Example:   two point charges closely spaced, opposite charge. Then φ ~ 1/r2 I think and E ~ 1/r3 and this is an electric dipole field. There is certainly SOME power that survives far away.

So, any argument based on Bm = 0 is just plain wrong!

Status:  I was hoping somehow the 2-conductor sum would magically make homo terms go away, but I was unable to show that.

Plan B. Still pondering the need to add homo terms.

I am now back to square 0. How does King know that BC's are automatically met? When I solve the single uniform round wire, I find that they are NOT automatically met and I have to add in some homo terms to make things work.

Reminder: Remember that King assumes in his (24) that all conductors have the same μ and that this is the same as the μ in the dielectric!  At least with my derivation of things, that is what seems to be the case. You would wonder then why he talks so much about hysteresis and such.

Is it possible that King's μ is a variable depending on region?  If that were the case, then the whole development would be different. I will run that right here. Here is (1.3.3) the starting point:

(2 - μ1ε1 ∂t2) A  = grad [μ1ε1 ∂tφ + divA ] - μ1J	.		// region 1		(1.3.3)

In this region, I use the King gauge and end up with

(2 - μ1ε1 ∂t2 - μ1σ1∂t) A = 0				// region 1			(1.3.19)

Fine. Now start over in region 2:

(2 - μ2ε2 ∂t2) A  = grad [μ2ε2 ∂tφ + divA ] - μ2J2			// region 2		(1.3.3)

Suppose I assume the King gauge in region 2 with region 2 parameters,     [ a rejected model ]

div A = - μ2ε2 ∂tφ - μ2σ2φ		// region 2

Then here is what happens:

(2 - μ2ε2 ∂t2) A  = grad [μ2ε2 ∂tφ + divA ] - μ2J2			// region 2		(1.3.3)

(2 - μ2ε2 ∂t2) A  = grad [μ2ε2 ∂tφ +{- μ2ε2 ∂tφ - μ2σ2φ} ] - μ2J2

(2 - μ2ε2 ∂t2) A  = grad [- μ2σ2φ} ] - μ2J2

(2 - μ2ε2 ∂t2) A  = - μ2σ2 grad φ ] - μ2J2

(2 - μ2ε2 ∂t2) A  = - μ2σ2 {-E - ∂tA} ] - μ2J2

Since inside a conductor in region 2, set E = 0 as per usual arguments, then have

(2 - μ2ε2 ∂t2) A  = - μ2σ2 { - ∂tA} ] - μ2J2

(2 - μ2ε2 ∂t2 - μ2σ2∂t) A  = - μ2J2

But I thought that you got that damping term only if you "absorb" the conduction current. Well, that was necessary in region 1 because i could not there say that E = 0. There I ended up like this for RHS.

= - μ1σ1 (-E -∂tA) -  μ1(σ1E)  .		 // from (1.3.1) and J = σ1E

= - μ1σ1 ( -∂tA) .

But in a conductor, maybe we can just set E = 0 as I have done above. Then we end up with this

(2 - μ1ε1 ∂t2 - μ1σ1∂t) A = 0				// region 1			(1.3.19)

(2 - μ2ε2 ∂t2 - μ2σ2∂t) A  = - μ2J2				// region 2

(2 - μ3ε3 ∂t2 - μ3σ3∂t) A  = - μ3J3				// region 3

The Helmholtz are then

(2+ β12) A = 0

(2+ β22) A = - μ2J2

(2+ β32) A = - μ3J3

I think all is OK to this point. But now I cannot make a unified region R since the three wave operators are different. [correct] In region 1 which is the main interest, there is no source, so there is no Helmholtz integral! There is then no way in the world I can arrive at his (24) using this interpretation of the gauge. The only way I know is to "do it my way" and then it works.

Here is my result:

(2 + β2)A =   - Σi=2N μiJi 	β2 = ω2μ ξ

Now if you make all the conductors the same μc you would have

(2 + β2)A =   - μc Σi=2N Ji 		β2 = ω2μ ξ

and then

A(x,ω) =   ∫[ Σi Ji(x',ω)] dV'

So maybe that is what King means. He means dielectric μ in β, and conductor μ out front. This μc is an overall factor, should not be too critical.

Now jump to my round wire analysis in section "Reminder of the Just-Fine...". Here is a summary

22D Az  =  0			region 1

22D Az  =  - μ2 Jcz			region 2

Az2 = (1/2π) ∫dV' [μ2Jcz(x')] ln(1/R)   =  W  = Helmholtz integral

W   = -(I/2π) μ2  		// by direct computation in App B I guess

Notice that this DOES automatically satisfy the boundary condition Az1(a) = Az2(a) without the addition of homo terms. And in fact Az1(a) =  -(I/2π) μ2 ln (a) at the boundary, from either side. But it does NOT satisfy the 2nd boundary condition on ∂rAz. If I want to satisfy BOTH these boundary conditions assuming no free surface current on the conductor, I must first add these homo solutions

A1homo = A lnr		// outside wire

A2homo = d			// inside wire

And then I match BC's and find that

A = -μ1 (I/2π)

d = (I/2π)(μ2-μ1)lna

In particular, outside the wire I had to add this homo solution

A1homo =  -μ1 (I/2π) lnr

Even if μ1 = μ2, I still have to add the above homo solution in the dielectric region 1.

[ I guess this is where I realized there is a slope condition on Az at a boundary ]

So where does my second BC come from? I will derive it right here in cyl coordinates.

curl A =  [ r-1∂θAz - ∂zAθ] +  [∂zAr - ∂rAz] +  [ r-1∂r(rAθ) - r-1∂θAr ]

Assume that A = Az(r,z)   does not depend on θ. Then

curl A =  [ r-1∂θAz] +  [- ∂rAz]    =   [- ∂rAz]   		so Bθ = - ∂rAz

Now from (1.1.44)

(1/μ2)Bθ2 -  (1/μ1)Bθ1	=  Kzfree

and then

(1/μ2)[ - ∂rAz]2 -  (1/μ1)[ - ∂rAz]1	=  Kzfree

(1/μ2)[ ∂rAz]2 -  (1/μ1)[ ∂rAz]1	=  - Kzfree			[ here it is ! ]

Thus I either have this BC or I have to have a free surface current. But that free surface current is not part of my prescribed current in the wire!  So there is no free current by assumption, so this BC has to be met. Then I have to add my homo term in the dielectric. Maybe I should write this up fully, but I have done it a lot of times.

You know, King's comments about BC's really only applies to the sum over all conductors, but you should be able to apply his (23) and (24) to a single conductor.  He says "arbitrary configurations of conductors". So I think he claims boundary conditions are "automatically met" for a single conductor.

He does restrict his (23) and (24) to the calculation of φ and A outside the conductors. So if you compute these solutions, you will get some value for Az as you approach any point on any conductor, so in that sense it meets this boundary condition. I don't like it!  I don't believe his claim. I have a clean counterexample. So maybe he just means by "boundary conditions" the fact that Az approaches some value on each of the plural conductors. That is true, but I argue that it is not the right value!

So how then does this "work out" in his first example?  He basically writes (23) and (24) as (10a) and (10b) for V and W where these are differences, and he has assumed no homo contribution here. Since I am in the DC limit I will grant that only his k0 survives.  In his (16) the first integral is well defined as in (19), but there is also a second integral shown. He works hard to argue that this integral goes away in the DC limit with order β2.

So to summarize:  if you take two round wires, and if you ignore homo solutions, and if you do the full 3D analysis which he does, and you deal with the differences V and W, then you get "the right answer" and no homo terms were needed.

Now suppose we were to add the homo terms anyway which I know are there for each wire.

Well, maybe I need to solve this problem using my 2D propagator stuff and see what happens in that case. That would be the n=2 problem for this issue! BUT, in the line problem, perhaps you don't have uniform current in each conductor, so my approach would be wrong?  But even he assumes b >> a and that currents are uniform.

Now what has happened to his transverse integral in (10)? That is all jammed into his I current.

So maybe I need to look at a fancier King example. He does that fat wires on page 24, but his approach here is to write my homo wave equation in region 1 which is (2 + β2)Az = 0. He then separates variables and ends up with 2D Helm equation which in my DC limit becomes a 2D Laplace equation and then goes and solves THAT for the fat wires. Nowhere to be seen are his (23) and (24) integrals!!!

Let's look now at his "3 phase" transmission line calculation on page 40. Here the three currents treated as complex in ω plane do in fact add up to 0 current if you add those three vectors at 120 degrees. And here he does use the Helmholtz integral for Az.

I just searched the book for "homogeneous" and could find no reference to my issue. Mostly the word refers to the nature of a medium.

OK, I see my puzzlement. King really does assume that μ is the same for dielectric and for conductors and that is the only way that his Az (23) or (24) work. In this case, his Az solution does "automatically" meet BC's at a boundary, because if μ1 = μ2 at the boundary, then there really IS no boundary and no reason for Az to have a slope break there. I think this is a strong limitation of his approach, but put that thought on hold for the moment.