Saved old Appendix H						PhL		11.14.13

I replaced this old App H with a pair of new appendices, App H and App I.  They subsume all the results developed in this old App H. So this is just for archive, can be tossed eventually.

## Appendix H : Calculation of 2[1/r] , 2[h(r)/r]  and 2[e-jkr/r], and 22D[ln(1/r)]

__________________________________________________________________________________

Theorem 1: 		-2[1/r] =  4πδ(r)							(H.1)

Proof: Let volume V be all of 3D space. Carve out from V a small spherical cavity of radius a centered at r = 0. If we call this spherical volume Va and then V' = V - Va is the original volume with the spherical cavity. In order to show that some function g(r) = δ(r), one has to show that

lima→0 ∫V' dV g(r) = 0									(H.2a)

lima→0 ∫Va dV g(r) = 1									(H.2b)

This is basically the definition of δ(r).  Since δ(r) has units L-3, g(r) has units L-3.

Our candidate function of interest is

g(r) = - (1/4π) 2[1/r]									(H.3)

Using 2 in spherical coordinates acting on a function of r,  one finds that, since ∂r(1) = 0,

2[1/r]   = (1/r2)∂r(r2∂r) [1/r]  = 0		r > 0						(H.4)

so that

g(r) = - (1/4π) 2[1/r]  =  0 			r > 0	. 					(H.5)

Thus, condition (H.2a) is trivially satisfied since  r > 0 everywhere in volume V'.

It remains to verify condition (H.2b). Consider the integral appearing in the left side of (H.2b)

∫Va dV g(r)  = - (1/4π) ∫Va dV 2[1/r]  =  - (1/4π) ∫Va dV   [1/r]   .			(H.6)

The divergence theorem says

∫V dV div F = ∫S dS  F									(H.7)

where V is any closed volume whose surface is S, and dS points out. Using

V = Va 	and 	F = [1/r] = ∂r(1/r) = -r-2

we find that

LHS (H.7)  = ∫Va dV div [1/r]  = ∫Va dV 2[1/r]  = ∫Va dV [-4πg(r)] = -4π ∫Va dV g(r)

RHS (H.7)	= ∫S dS  [1/r] =  ∫dΩ [a2 ]  [1/r]|r=a = ∫dΩ[a2 ] [-a-2] = -4π

which tells us that ∫Va dV g(r) = 1 for any a. Thus,

lima→0 ∫Va dV g(r) = 1

and we have then verified (H.2b).  Therefore we conclude that

- (1/4π)2[1/r]  = δ(r)									(H.8)

or

2[1/r]  = - 4πδ(r)

which is (H.1).											QED

__________________________________________________________________________________

Theorem 2: 		2[h(r)/r]  = - 4π h(0) δ(r)  + h"(r)/ r					(H.9)

Proof:  Start with this vector identity

2(φψ) = φ2ψ + ψ2φ + 2 φ  ψ							(H.10)

so that

2(h r-1) = h2(r-1) + r-12h  + 2  h  (r-1)

= - h 4π δ(r) + r-12h + 2 [ h'    (-r-2)

= - 4π h(0) δ(r) + r-12h  - 2 r-2 h'(r)							(H.11)

Algebra shows that,

2h =  (1/r2)∂r(r2∂r)h(r) =  h"(r) + (2/r)h'(r)							(H.12)

so then

2(h r-1) = - 4π h(0) δ(r)  + r-1 [h"(r) + (2/r)h'(r) ] - 2 r-2 h'(r)

= - 4π h(0) δ(r)  + h"(r)/ r

which is the claim of (H.9). 									QED

__________________________________________________________________________________

Example of Theorem 2:  h(r) = e-jkr

h = e-jkr		h(0) = 1		h' = -jk e-jkr 	 	h" = -k2 e-jkr

2(e-jkr/r) = - 4π 1 δ(r)  + [-k2 e-jkr ] / r

= -4πδ(r) - k2(e-jkr/r)

Thus,

( 2+k2) (e-jkr/r) = - 4πδ(r)								(H.13)

or

- (2+k2) (e-jkr/4πr)  = δ(r)								(H.14)

If we translate the origin from 0 to x' and define R = |r - x'|  and rename r to be x, (H.13) becomes

- (2+k2) (e-jkR/4πR)  = δ(x-x')								(H.15)

Thus we may identify e-jkR/4πR as the Helmholtz equation Green's Function. In the case k = 0 we get

- 2(1/4πR)  = δ(x-x')									(H.16)

and we may identify (1/4πR) as the Poisson equation Green's Function as shown in (A.0.3) and (A.0.4).

__________________________________________________________________________________

Theorem 3: 		2[ln(1/r)] = - 2πδ(r)	 in two dimensions				(H.1)'

Our 2D proof here will be an "copy, paste and edit" version of the 3D proof of Theorem 1. We shall use the word "volume" with symbol V to denote area. A "spherical cavity" now means a circular hole. It should be understood that 2 = ∂x2+∂y2 and δ(r)= δ(x)δ(y). We indicate all edits in red and equation numbers are the same as for Theorem 1 with a prime added.

Proof: Let volume V be all of 2D space. Carve out from V a small spherical cavity of radius a centered at r = 0. If we call this spherical volume Va and then V' = V - Va is the original volume with the spherical cavity. In order to show that some function g(r) = δ(r), one has to show that

lima→0 ∫V' dV g(r) = 0									(H.2a)'

lima→0 ∫Va dV g(r) = 1									(H.2b)'

This is basically the definition of δ(r).  Since δ(r) has units L-2, g(r) has units L-2.

Comment:  When any differential operator like  or 2 is applied to ln(r0/r), the result is independent of r0 so we can always take r0 = 1. For example, ∂x [ln(r0/r)] = ∂x [ lnr0 + ln(1/r)]  = ∂x ln(1/r). In what follows, ln(r) and ln(1/r) are always acted upon by differential operators, so we can interpret these objects as dimensionless quantities ln(r/r0) and ln(r0/r). Then it is clear below that dim [g(r)]  = L-2.

Our candidate function of interest is

g(r) = - (1/2π) 2[ln(1/r)]	= +(1/2π) 2 [ ln(r)] 						(H.3)'

Using 2 in polar coordinates acting on a function of r,  one finds that, since ∂r(1) = 0,

2[ln(r)]   = (1/r)∂r(r∂r) [ln(r)]  = 0		r > 0						(H.4)'

so that

g(r) =  - (1/2π) 2[ln(1/r)]  =  0 		r > 0	.	 				(H.5)'

Thus, condition (H.2a)' is trivially satisfied since  r > 0 everywhere in volume V'.

It remains to verify condition (H.2b)'. Consider the integral appearing in the left side of (H.2b)'

∫Va dV g(r)  = - (1/2π) ∫Va dV 2[ln(1/r)]  =  - (1/2π) ∫Va dV   [ln(1/r)]   .		(H.6)'

The divergence theorem says

∫V dV div F = ∫S dS  F									(H.7)'

where V is any closed volume whose surface is S, and dS points out. Using

V = Va 	and 	F = [ln(1/r)] = ∂r(ln(1/r))  =  - ∂r(lnr)  =  [ -r-1]

we find that [ here dS  = dS   where dS = adθ = a piece of circumference of the circle bounding Va ]

LHS (H.7)  = ∫Va dV div [ln(1/r)]  = ∫Va dV 2[ln(1/r)]  = ∫Va dV [-2πg(r)] = -2π ∫Va dV g(r)

RHS (H.7)	= ∫S dS  [ ln(1/r)] =  ∫dθ [a ]  [ ln(1/r)]|r=a = ∫dθ [a ] [-a-1] = -2π

which tells us that ∫Va dV g(r) = 1 for any a. Thus,

lima→0 ∫Va dV g(r) = 1

and we have then verified (H.2b)'.  Therefore we conclude that

- (1/2π)2[ln(1/r)]  = δ(r)									(H.8)'

or

2[ln(1/r)]  = - 2πδ(r)

which is (H.1)'.											QED