Paradox with Appendix P Section 8 field plot figures		PhL		5.5.14

I don't think the red lines should be perfectly straight inside the wires in these plots, so I think something is wrong. [ it turns out the just look straight but are not exactly straight, so no paradox ]

First, here is the math from "the Surface Currents issue..doc" :

H1(r1) = (I/2π)[ θ(r1>a1) (1/r1)  + θ(a1>r1)  (r1/a12)]

H2(r2) = -(I/2π)[ θ(r2>a2) (2/r2)  + θ(a2>r2)  (r2/a22)]

Hx = - (y/r1)H1(r1) - (y/r2) H2(r2)

Hy = (x/r1) H1(r1) + ((x-b)/r2) H2(r2)

If I look only at Hy we get

Hy = (x/r1) H1(r1) + ((x-b)/r2) H2(r2)

=  (x/r1) [(I/2π)[ θ(r1>a1) (1/r1)  + θ(a1>r1)  (r1/a12)]]

+  ((x-b)/r2) [-(I/2π)[ θ(r2>a2) (2/r2)  + θ(a2>r2)  (r2/a22)]]

Now suppose we are close to the center of the left wire and we select using the Heavisides. Then we select the terms I show above as red

Hy 	=  (x/r1) [(I/2π)[ θ(a1>r1)  (r1/a12)]]

+  ((x-b)/r2) [-(I/2π)[ θ(r2>a2) (2/r2)]]

= (I/2π)  [(x/r1) (r1/a12)  -   ((x-b)/r2) (2/r2)]

= (I/2π)  [x/a12  -   2 ((x-b)/r22)]			r22 = (x-b)2 + y2

Now suppose I look only at y = 0. This then says

Hy(x,y=0) 	= (I/2π)  [x/a12  -   2 ((x-b)/(x-b)2)]

=  (I/2π)  [x/a12  -  2/(x-b)]

This function is NOT linear in x, but my plots show things as linear in x close to wire 1.

This is the paradox. With a1 = 1/2 and b = 2 this says

=  (I/2π)  [4x   -  2/(x-3)]

But OK, here is a Maple plot of this function:

The first term is so dominant