Better expression for R and Li					PhL		9.6.14

This is the original doc version. It has the same problem as version 2, so it is irrelevant! R in k(ω) is not the bulk R, it is real part of Zs.

My current Appendix Q model with simple Heaviside functions seems too crude. In that model, C = C and G = expression valid for all ω. The problem is with Li and R. I think I can come up with a better and smoother model for these two parameters based on physics.

At high ω, the current flows in a sheaf and I have expressions for Li and R for such a sheaf. In lines doc this is treated in App C and App K and App R for Belden. See App R section (f).

I will now " try a few things" in hopes of coming up with something better.

Let's start at relatively high ω so we have this picture

where we see the annulus and a crude plot of Jz versus radius. Our model here from (D.10.11) for Jz in a given partial wave m is,

Jz(r,m)  = -(j/2) σ ηm I Rdc (aβ)  e(1+j)(r-a)/δ

Also from (D.10.15),

Ez(r,θ)  =  - (jω/σ) e(1+j)(r-a)/δ  n(θ) (β/k)

Jz(r,θ)  =  - jωe(1+j)(r-a)/δ  n(θ) (β/k)

In a separate file (App D filed) I just verified that Jz integrates to I as long as a/δ >> 1. If this last is not true, the erf appears in the integral.

Of course I claim to have this more general answer, at least in partial waves,

Jz(r,m)  = σ (1/4) ηm I Rdc (aβ') fm(r) 	Note that Jz(r,-m) = Jz(r,m)  for Ch 6 case

Jz(r,θ) = Σm=-∞∞ Jz(r,m) ejmθ

= Jz(r,0) + 2 σ (1/4) I Rdc (aβ')  Σm=1∞ ηm fm(r) cos(mθ)

But maybe I can just assume a coaxial cable so we have a symmetric in θ situation. Then Jz is what I computed in Ch 2. Here that is just

Jz(r,θ) = Jz(r,0) = σ (1/4) I Rdc (aβ') f0(r)  = σ (1/4) I Rdc (aβ') f0(r)

= σ (1/4) I Rdc (aβ') 2 J0(x)/J1(xa)  = σ (1/2) I Rdc xa J0(x)/J1(xa)

= (1/2) I (1/πa) (β) J0(x)/J1(xa)	 β ≠ β'   =   (I/2πa) (β) J0(x)/J1(xa)

which agrees with (2.2.30).

So let's perhaps simplify the problem and just work on coaxial cable center conductor. We then have

Jz(r,θ)  =  (I/2πa) (β) J0(β'r)/J1(β'a)		β'2 = β2- k2

and perhaps I could add a factor n(θ) / <n(θ)> = n(θ)/ N0 to simulate the θ dependence.

So here then is the question:  given the above Jz function, what can I say about R and Li for the center conductor?  Since k2 appears here, we have a somewhat circular situation since I am trying to create a model from which I can compute k2!!  I think I will have to make the ansatz that β' ≈ β. I recently studied this question in " Appendix M rewrite done Aug 19, 2014 original.doc" and it seems reasonable there based on my simple 2-phase model for k. So let's just assume that

Jz(r)  =  (I/2πa) (β) J0(βr)/J1(βa)		ansatz

From this Jz(r) function, I should be able to compute both R and Li at all ω !

Li = μi ∫in dS (Hi/I)2									(C.3.7)

The magnetic field Hi at radius r is determined by

2πr H(r) = Ienc  = !Syntax Error, Ir dr!Syntax Error, Idθ Jz(r)  = 2π !Syntax Error, Ir dr Jz(r)

= 2π  !Syntax Error, Ir dr J0(βr)   = !Syntax Error, Ir dr J0(βr)

Now let

x = βr	dx = βdr	xdx = β2rdr

so that

2πr H(r)  =   !Syntax Error, Ixdx/β2 J0(x)  =  !Syntax Error, IxJ0(x) dx

This I know is a doable integral.  GR7 p 630 says

so the integral is just [xJ1(x)]|βr0  = (βr)J1(βr), so we find that

2πr H(r)  =    (βr)J1(βr)  = I (r/a) J1(βr)/ J1(βa)

which is very simple. When r = a you get I as expected. So:

H(r) = I (1/2πa) J1(βr)/ J1(βa)			amp/m

Then I can compute the internal inductance of the center conductor this way

2Li = μ ∫in dS |H/I|2 		// correction made for complex fields, see below

= μ 2π !Syntax Error, Ir dr  |(1/2πa) J1(βr)/ J1(βa)|2

Do a check right here:   For small ω,  J1(z) = z/2  so

2Li = μ 2π (1/2πa)2 !Syntax Error, Ir dr  (r/a)2 = μ 2π (1/4π2a2) a-2 (1/4)a4  = μ/8π

But this seems wrong by factor of 2. So get this cleaned up before continuing.

= μ 2π (1/2πa)2 |1/ J1(βa)|2 !Syntax Error, Ir dr |J1(βr)|2

= μ 2π (1/4π2a2) |1/ J1(βa)|2 !Syntax Error, Ir dr |J1(βr)|2

= μ (1/2πa2) |1/ J1(xa)|2 !Syntax Error, Ir dr |J1(βr)|2

= μ (1/2πa2) |1/ J1(xa)|2  K

The integral here is

K = !Syntax Error, Ir dr |J1(βr)|2 = !Syntax Error, I xdx/β2 * |J1(x)|2   = (1/β2) !Syntax Error, I xdx  |J1(x)|2

Check:  For small ω, have J1(βr) ≈ (βr/2) and then K = !Syntax Error, Ir dr (|β|r/2)2  = |β2|/4 * !Syntax Error, Ir3 dr

= |β2|/4 * (1/4) a4   = |β2| a4 .

Then we get

2Li =  μ (1/2πa2) (2/|β|a)2 |β2| a4 = μ (1/2π) (2)2  = 2μ/π 	=>	Li = μ/π  = wrong

STOP. With first = you can see that K is a real number, but this fact seems lost in the next form. Let's just leave it as is and write

K = !Syntax Error, Ir dr |J1(βr)|2  = !Syntax Error, Ir dr J1(βr) J1*(βr)

Side Note:  If β is very small, we get K ≈ a2/2 and then

2Li =  μ (1/2πa2)(2/|xa|)  a2/2  = μ (1/2πa2)(2/a|β|)  a2/2  = μ (1/4π)(2/a|β|)  = wrong!!

Now I think we can claim from the series form of J

that Jν*(z) = Jν(z*). I thought this is "real analytic" but I see from the web that is a not the right meaning of real analytic, there seems to be no name for this thing. If the power series has real coefficients, then you conclude that Jν*(z) = Jν(z*), so fine. We then have

K = !Syntax Error, Ir dr J1(βr) J1(β*r)

Luckily I can evaluate this integral using

where I will take the first form. Then

K = !Syntax Error, Ix dx J1(β*x) J1(βx)

=  [ β* x J1(β*x)J0(βx) - βx J0(β*x)J1(βx) ] |a0

The zero end evaluation gives nothing since J1 = 0 there, so we then have

K =  [ β* a J1(β*a)J0(βa) - βa J0(β*a)J1(βa) ]

=  [ xa*J1(xa*)J0(xa) - xa J0(xa*)J1(xa) ]

Now we know that  (z2)* = (z*)2 . Reverse order in both top and bottom to get

K =   [xa J0(xa*)J1(xa)  - c.c. ]   =  2j Im[xa J0(xa*)J1(xa)]

=   Im[xa J0(xa*)J1(xa)]   = real!

Just verifying that K is real.

Side Note:  If β is very small, we get K =  Im[(xa2)/2] =  Im[β2a2/2]  = a2/2 so OK.

But I want to write this a different way. Note that

β2 = -jωμσ		=>	Imβ2 = -ωμσ

Go back so that

K =   [xa J0(xa*)J1(xa)  - c.c. ] 		dim K = m2

=   [xa J0(xa*)J1(xa)  - c.c. ]

=  [xa J0(xa*)J1(xa)  - xa* J0(xa)J1(xa*) ]

=  [xa J0*(xa)J1(xa)  - xa* J0(xa)J1*(xa)]

So we then have

2Li = μ (1/2πa2) |1/ J1(xa)|2  K

= (μ/2πa2)   [xa J0*(xa)J1(xa)  - xa* J0(xa) J1*(xa)]

=  (μ/2πa2)  [xa J0*(xa)/J1*(xa)  - xa* J0(xa)/J1(xa)]

=  (1/4πa2) [   -  ]

=  (j/4πa2σω) [   -  ]

and finally here is my result:

Li =    [   -  ]

But OK, now combine the two terms to get 2jIm(..) to get

Li =    2j Im [  ]	   = -   Im [  ]	// real at least

Now evaluate this at small β meaning small xa to get

J0 = 1		J1 = xa/2

Im [  ]  = Im [  ]  = 2  Im  [  ]  =  2 Im []

Now write β  ≡ ej3π/4  from (2.2.12)  to get

= 2 Im []  = 2 Im[e+j3π/2] = 2 Im[-j]  = -2

So our low ω limit is then

Li = -  [ -2] = wrong!!!

***********************

So we then have

Li = μ 2π (1/2πa)2 |1/ J1(βa)|2 * (1/β2) !Syntax Error, I xdx |J1(x)|2

=  !Syntax Error, I xdx |J1(x)|2  =  !Syntax Error, I xdx |J1(x)|2

Maple cannot do this integral. But GR7 can using

which says

!Syntax Error, I xdx [J1(x)]2  =  [ (x2/2) {  [J1(x)]2 - J0(x)J2(x) }|βa0

= (β2a2/2) {  [J1(βa)]2 - J0(βa)J2(βa) }

= (xa2/2) {  [J1(xa)]2 - J0(xa)J2(xa) }

and so

Li =   (xa2/2) {  [J1(xa)]2 - J0(xa)J2(xa) }

=  {  [J1(xa)]2 - J0(xa)J2(xa) }}

=  ( 1 -   )		xa = βa

Check some limits here. At low ω, xa is small and we use Jn(x) = (x/2)n / n! so

J0 = 1		J1 = xa/2		J2 = (xa/2)2/2

so then

Li =  ( 1 - )  =  ( 1 - )  = μ/8π   hurray!!

Fine, but Li is complex, so I think I need to fix something in Appendix C interpretation. There I write

u(r) = (1/2) μ H(r)2.										(C.3.1)

So what does it mean that the energy density has an r-dependent phase?  I suspect this should say

u(r) = (1/2) μ |H(r)|2.									(C.3.1)

My appendix is only for DC applications. Jackson says uem = (ED + BH)/2 . But if we have monochrome fields, this usual thing happens

H = H1cos(ωt)

2<u>/μ  = < HH> = ∫ dt [H1cos(ωt)]2 = (1/2) H1  H1

And if

H = H1ejωt

then

H  H*  =  H1  H1  = 4 <u>/μ

so the rule is this

<u> = (1/4) μ H  H*

so you add a * to one of the fields, and you add an extra overall 1/2. To quote from Jackson

so that is an example of this extra factor of 1/2.  So I will now REDO the above calculation