This is the Title						PhL		3.26.05

This has the field continuity conditions in perp and parallel notation. First part is incorporated into Ch 1 of lines doc. I decided on a t and n notation instead.

If the two media are conducting dielectrics with Ohm's law Jc = σE, we can apply continuity (**) to our tiny box to find that

div Jc =  - ∂tρfree	 		-∂t[∫V ρfree dV]  =  ∫S Jc  dA			(1.1.16)

so that

-∂tnfree = [Jn1- Jn2]  = σ1En1 - σ2En2

or for monochrome time dependence

-jω nfree = σ1En1 - σ2En2 .

Recall now from *** that

nfree = ε1En1- ε2En2

Adding the last equation to 1/jω times the previous equation gives

0 = [ε1 + σ1/jω] En1  - [ε2 + σ2/jω]En2

In terms of the complex dielectric constants ξi this says that 0 = ξ1En1 - ξ2En2 so that

ξ1En1 = ξ2En2

In the limit that say medium 2 becomes a perfect conductor, ξ2 ≈ σ2/jω → ∞ and En2 → 0, but the product is maintained equal to ξ1En1 .

But there is no free charge in such dielectrics, nor any at the boundary between them unless supplied by a very contrived apparatus, because any such free charge would leak away into the dielectrics. If one of the dielectrics is a "conductor",

There will be polarization charge npol at the boundary due to "differential displacement" of the two dielectrics there.

********************** saving things here ************

E||(1) = E||(2)		or		(1/ε1)D||(1) = (1/ε2)D||(2)			(1.1.20)

H||(1) = H||(2)		or  		(1/μ1)B||(1) = (1/μ2)B||(2) 			(1.1.21)

[D(1) - D(2)]  = nfree	or		[ε1E(1) - ε2E(2)]  = nfree			(1.1.22)

B(1) = B(2)		or		μ1H(1) = μ2H(2) 				(1.1.23)

Rules for continuity of parallel and normal fields at a boundary: 				(1.2.24)

E||(1) = E||(2)		or		(1/ε1)D||(1) = (1/ε2)D||(2)	(1.1.20)

H||(1) = H||(2)		or  		(1/μ1)B||(1) = (1/μ2)B||(2)	(1.1.21)

[D(1) - D(2)]  = n	or		[ε1E(1) - ε2E(2)]  = n	(1.1.22)

μ1H(1) = μ2H(2)		or 		B(1) = B(2)			(1.1.23)