New Retarded path not taken					PhL		9.19.13

I think this is my working doc for updating this section. It is now installed.

### 1.4 Retarded Solutions: a path not taken

In a medium with σ = 0 and in which μ and ε are time-independent, equations (1.3.7) and (1.3.8) apply,

2 -  με ∂t2)A  = - μJT									(1.3.7)

(2 -  με ∂t2)φ  = - (1/ε)ρT									(1.3.8)

One approach to solving these equations for A and φ is the method of retarded solutions. Although we shall not use this approach, we outline the method here. We seek to solve an equation of this form

(2 -  με ∂t2) u  = - f									(1.4.1)

where for example in (1.3.8) we have u = φ and f = ρT/ε.  Since με = 1/v2 where v is the wave velocity in the medium, write (1.4.1) as

(∂t2 - v22) u  =  v2f

or

u  = f	where     ≡ ∂t2 - 2 .							(1.4.2)

This last equation is similar to (A.7.2) of Appendix A and we solve it in the same manner. Define a Green's function g as the solution of

v2  g(x,t; x',t') = δ(x-x')δ(t-t')  .								(1.4.3)

As Appendix A section 7 shows, the solution is given by

v2 g(x,t; x',t')  =   (1/4πR)δ(t-t'-R/v)	   with   R = |x-x'|   .					(1.4.4)

Jackson (6.41) and (6.44) uses G(+) = 4πv2g with v = c and refers to the solution as a "retarded Green function". See also Stakgold references in Appendix A. Then the solution to (1.4.1) is

u(x,t)  =  ∫d3x' ∫dt' v2 g(x,t; x',t') f(x',t')

as can be verified by applying  to both sides and making use of (1.4.3). Inserting (1.4.4) then gives

u(x,t)   =  ∫d3x' ∫dt'  (1/4πR)δ(t-t'-R/v) f(x',t')

=  ∫d3x' (1/4πR) f(x',t-R/v)

=   ∫d3x'  								(1.4.5)

Thus, the solutions to (1.3.7) and (1.3.8) are

A(x,t)  =   ∫d3x'  								(1.4.6)

φ(x,t)  =   ∫d3x'  								(1.4.7)

The potentials at time t are generated by the values the sources had at time t - R/v since the influence of the sources travels at finite velocity v through the medium. Compare (1.4.7) to (A.0.2) which is the solution to the electrostatic Poisson equation.

With the medium σ loss term present, and with frequency dependent (hence time dependent) μ and ε, it is better to work in the frequency domain instead of the time domain. Thus, although they are interesting, we shall not use these retarded solutions.