Matick Section 4.5						PhL		1.9.14

Since I mention this section in my original lines doc, I might as well read it (probably for the first time).

[p 108] The stripline is drawn, width W and separation S with W >> S. What I usually call the z direction (wave direction) he has as his x direction.

[ p 109]  This is all statement of Maxwell's equations.

[ p 110]  He obtains the vector Helmholtz equation for the E field, same as mine.  His E points only across the gap, while B is in the W direction. His equation (4.50) is strange since the LHS is a vector which is 2E  whereas the rest of the equation is scalar. I have fixed this up.

[p 111-113] He assumes the form (4-52) for his longitudinal E field where γ and Γ1 are TBD.  He then just goes on and on for several pages. He is matching Ez values at the conductor surface. He then gets a transcendental equation for constant γ in (4-66).

[p 114-116] He discovers the skin effect δ value. He computes inductance L internal and external.

The following is now in lines doc Chapter 5:

In the 12-page Section 4.5 of his book Matick studies a lossy-transmission line in the simplest possible case which is a stripline geometry whose gap S is small compared to the width and whose metal strips are much thicker than the skin depth δ. His parameter γ is related to our parameter k by γ = jk, and his longitudinal direction is x instead of our z. He ends up with a transcendental "eigenvalue equation" (4-66) for γ, but if loss is very small, he can approximately solve for γ with these results

Im(γ)  =  β(1+δ/2S)		Re(γ) = β (δ/2S)			// Matick (4-75,76,77)

which with γ = jk we translate to

Im(k) = - Re(γ) = - β (δ/2S)

Re(k) = Im(γ) =  β(1+δ/2S)

k = β(1+δ/2S) -j β (δ/2S)  =  β - [-(δ/2S) + j(δ/2S)]

so

Δ = -(δ/2S) + j(δ/2S) .

Thus, for such a stripline transmission line, the longitudinal dependence of all functions has this form,

q(z,t) = ej(ωt-[β-Re(Δ)]z)  e–Im(Δ)z

= ej(ωt-[β+δ/2S]z)  e–(δ/2S)z

which shows the exponential loss factor and an increased wavenumber β+δ/2S which corresponds to a decreased wavelength λ and a decreased wave velocity v = ω/k = ωλ/2π = fλ, the "drag effect".

Matick has an errata in this section which is a bit confusing, so we repair it right here. His equation (4-50) should read

2E = ( + )   + ( + )   = (jωμσ - ω2με)(Ex + Ez)		Matick (4-50)