This is the Title						PhL		3.26.05

This has been installed into lines doc.

#### (f) Time domain equations

The results above are all stated in the frequency domain, but it is a simple matter to convert them to the time domain using jω  ↔  ∂t.  One then makes these replacements

z  = R+jωL 		→	L∂t + R

y  = G+jωC 	→	C∂t + G

zy = (R+jωL)( G+jωC)   →	   (R + L∂t)( G + C∂t)  = LC∂t2 + (LG+RC)∂t + RG		(4.11.38)

Here then are selected equations and their translations to the time domain:

Transmission Line Equations (4.11.14b) :			[ coupled first order PDE's]

∂zV =  - z i 			∂zV(z,t) =  - L∂ti(z,t)  - LR i(z,t)

∂z i  =  -y V			∂z i(z,t)  = - C∂tV(z,t) - CGV(z,t)				(4.11.39)

Transmission Line Wave Equations (4.11.15) 			[ damped wave equations ]

( ∂z2 - zy) V(z) = 0		   [ ∂z2 - LC ∂t2 - (LG+RC)∂t - RG] V(z,t) = 0

( ∂z2 - zy) i(z) = 0		   [ ∂z2 - LC ∂t2 - (LG+RC)∂t - RG] i(z,t) = 0			(4.11.40)

If we set the loss factors R and G both to 0 these equations become

∂zV =  -L∂ti		[ ∂z2 - LC ∂t2] V(z,t) = 0	[ undamped wave equations]

∂z i  =  -C∂tV		[ ∂z2 - LC ∂t2] i(z,t) = 0		// lossless			(4.11.41)

At large ω one has L ≈ Le (note 1 above) and since (4.11.28) says LeC = με  = 1/v2 we conclude that the factor LC appearing in the above wave equations is 1/v2 where v is the dielectric wave velocity.

The various transmission line equations shown above in the time domain are often referred to as the telegraph (telegrapher, telegrapher's) equations.