Lamp Code Speaker Transmission Line			PhL		6.10.14

See comments at the end. For those reasons, I did not try to include this in lines doc!

So ε = 5 or ε = 6 is reasonable!

lamp wire 2:		d= 1 mm		b = 3.0 mm		PVC

Set G = 0.  First compute R from R = 1/(πa2σ) :

which is maybe 1 ohm for 45 meters of wire??  Seems low? Well agrees with "std compu power cables" so OK.

C = 2πε

First get ξ for the left wire:

Fine. Now here is C:

So C = 79 pF per meter!

What about L?? We know that

Le =   K

C = 4πε/K

LeC = με  = μ0ε0εrel   = c-2 εrel

so we are talking 0.7 μH/meter which seems OK. This is 700 nH/meter.

Lets throw in Lint = 2Li  = μ0/4π = 100 nH/m.

So ballpark maybe L = 800 nH/m.  So we then have

G = 0

R = .04 Ω/m   for the sum of both wires

C =  79 x 10-12 F/m

L  = 800 x 10-9 H/m for the sum of both wires

By one estimate we have

So I am getting 100 Ω as Z0 as long as

ωL >> R

2πf >> (R/L)

f > (R/2πL);

Ouch. So at 10 KHz maybe we get this real cable impedance.

What is δ?  δ ≡   = [ 2/2πfμσ]1/2 =  [ 1/πfμσ]1/2

At 1 KHz I get δ = 2 mm while a = 0.5 mm, so not in skin limit.

At 4 Khz I get δ = 1 mm while a = 0.5 mm.

So I have several problems with this little calculation:

(1) the cable impedance is coming out at 100 ohms resistive for f >> 4.4 KHz

which does not match an 8 ohm speaker. At lower f the Z0 is not resistive. People would be interested in low f for bass where power is high, but there we don't have a well-terminated transmission line.

(2) probably a speaker is an inductive load in general anyway and varying with ω.

(3) this is probably a terrible example of a transmission line and I would do well NOT to include it!!!