Archive this rejected section of Chapter 7			retired on 9/21/14

### 7.4 A Second Sign of Trouble: Infinite B fields as ω→0

In Appendix D we compute the E fields inside a round wire as outlined above, then the corresponding B fields are computed from the Maxwell curl E equation,

curl E = -jωB	=>	B = (j/ω) curl E

In the partial wave m space, and in cylindrical coordinates, this Maxwell equation reads,

B  =  (j/ω) {   [ r-1jmEz+ jkEθ] +  [-jkEr - ∂rEz] +  [ r-1∂r(rEθ) - r-1jmEr ] }

or

Br(r,m) =  (j/ω) [ r-1jmEz(r,m)+ jkEθ(r,m)]

Bθ(r,m) =  (j/ω) [-jkEr(r,m) - ∂rEz(r,m)]

Bz(r,m) =  (j/ω) [r-1∂r(rEθ(r,m)) - r-1jmEr(r,m)]

where as usual we have replaced ∂θ → jm and ∂z→ -jk.

We are interested in examining these B fields as ω→0. Near this limit, we know the behavior of k from Appendix Q.

We first assume that Gdc = 0 (vacuum dielectric) which means ωd = 0 in Appendix Q. We then find --  assuming that the equation k = k(ω)  ≡  -j  is valid in this limit -- that

Fact 4:  The small ω limit for k(ω), assuming ωd = 0, is given by			           (Q.4.9)

Re(k) ≈  +  ( 1 -  tanL/2)  + O(ω3/2)

Im(k) ≈   -  ( 1 + tanL/2)  + O(ω3/2)

where Rdc is the total resistance per length of both transmission line conductors. Ignoring the small loss tangent, this says

k(ω)  =  (1 - j)  =  e-jπ/4

k2 = -j RdcC ω .

Meanwhile, we know from ** that β2 = -jμσω. It then follows that

β'2 ≡ β2 - k2  →  -jμσω  + j RdcC ω  = j [-μσ + RdcC] ω

As ω→ 0, β' becomes very small, and xa ≡  β'a  and x ≡ β'r  (for r ≤ a) become << 1. We can therefore use the E field expressions given in (D.11.7) ,

Ez(r,m)  = (1/2) ηm I R (r/a)|m| (|m|+1)

Er(r,m)  = (j/4) ηm I R (ak) [(r/a)|m|+1 +  (r/a)|m|-1]				          (D.11.7)

Eθ(r,m) = (1/4) ηm I R (ak) [(r/a)|m|+1 -   (r/a)|m|-1]

Ez(r,0)  = I R 			R = 1/(σπa2)

Er(r,0)  = (j/2) I R (ak) (r/a)

Eθ(r,0) = 0								// low ω E fields

Whereas Rdc is the DC resistance of both conductors, R is the DC resistance just of the round wire.

In these expressions, we may write, using ** and **,

I = 2πa(ω/k)N0  =  2πa(ω/k){q/2πa} = q (ω/k)  = CV (ω/k)

But  for small ω,

(ω/k) = ω / [ e-jπ/4 ]  = ejπ/4

so

I = CV (ω/k) =   { ejπ/4  } V

This implies that Z0-1 = I/V = { ejπ/4 }, something we can quickly verify (as ω→ 0) :

Z0 =    =

=>	Z0-1 =   = ejπ/4  .

Notice that I → constant *   and Z0 → constant /  .  In the final ω→0 limit,  I → 0 and Z0 → ∞, but we are interested in what happens very close to that limit.

We can rewrite our E fields for small ω using I = CV (ω/k). On 3 lines we replace (ω/k) (ak) = a, while on two lines we use C (ω/k) = ejπ/4  . The resulting small ω fields are

Ez(r,m)  = (1/2) ηm V ejπ/4   R (r/a)|m| (|m|+1)

Er(r,m)  = (j/4) ηm CV (ω) R (a) [(r/a)|m|+1 +  (r/a)|m|-1]				          (D.11.7)

Eθ(r,m) = (1/4) ηm CV (ω) R (a) [(r/a)|m|+1 -   (r/a)|m|-1]

Ez(r,0)  = V ejπ/4   R

Er(r,0)  = (j/2) CV (ω) R (a) (r/a)

Eθ(r,0) = 0								// low ω E fields

In preparation for evaluating the B field, we compute these quantities

∂rEz(r,0)   = 0									m=0

∂rEz(r,m)	 =  (1/2) ηm V ejπ/4   R |m| (r/a)|m|-1 (1/a) (|m|+1)		m>0

r ∂r(rEθ(r,0))  = 0

r ∂r(rEθ(r,m)) = r ∂r [(1/4) ηm CV (ω) R (a) [(r/a)|m|+2 -   (r/a)|m|]]			m=0

= r (1/4) ηm CV (ω) R (a) {  (|m|+2) (r/a)|m|+1 - (|m|) (r/a)|m|-1 } (1/a)

=  (1/4) ηm CV (ω) R (a) {  (|m|+2) (r/a)|m|+1 - (|m|) (r/a)|m|-1 } (r/a)

=  (1/4) ηm CV (ω) R (a) {  (|m|+2) (r/a)|m|+2 - (|m|) (r/a)|m| }			m > 0

As a space-saving measure, we redisplay the above fields showing only the ω dependence, making up dummy names for the combination of factors.

Ez(r,m)  = Am

Er(r,m)  = Bm ω			          (D.11.7)

Eθ(r,m)  = Cm ω

Ez(r,0)  = A0

Er(r,0)  = B0 ω

Eθ(r,0) = 0								// low ω E fields

which we simplify to read

Ez(r,m)  = Am

Er(r,m)  = Bm ω			// for all m, and C0 = 0

Eθ(r,m)  = Cm ω

We may now insert these expressions for the small-ω fields into our B = (j/ω) curl E equations above,

Br(r,m) =  (j/ω) [ r-1jmEz(r,m)+ jkEθ(r,m)]

Bθ(r,m) =  (j/ω) [-jkEr(r,m) - ∂rEz(r,m)]

Bz(r,m) =  (j/ω) [r-1∂r(rEθ(r,m)) - r-1jmEr(r,m)]

For Br

Br(r,m) =  (j/ω) [r-1jm (1/2) ηm V ejπ/4   R (r/a)|m| (|m|+1)

The E fields computed in Appendix D may be written as

Second summary of the E field solutions : 		Rdc = 	β'2 = β2 - k2	         (D.2.33)

Ez(r,m)  = (1/4) ηm I Rdc (aβ') fm		fm  = [  -   ]		x =  β'r

Er(r,m)  = (j/4) ηm I Rdc (ak) gm		gm = [   +  ]		xa = β'a

Eθ(r,m) = (1/4) ηm I Rdc (ak) hm		hm = [  -  ]

This is too messy, I want another way to show this problem.