A Case Study							PhL		11.13.19

Fully reviewed.

Here I am going to look at the A solution for the round wire with uniform current in two ways, which I will call the Problem I Way and the Problem II Way. This is all done at ω = 0 and in the 2D way

Problem I Way.  [ this method is supportable ] The problem is described separately for regions 1 and 2, there is no common PDE, there is no image current on the boundary.

(2 + β12) AI  =  0 				x in region 1

(2 + β12) AI  =  - μ2 Jc2			x in region 2

(AzI)2  = (AzI)1

(1/μ2) (∂nAzI)2 =  (1/μ1) (∂nAzI)1

To solve this problem I make these assumptions:

1. The solution in region 2 is the particular solution plus a constant d. Thus

AI,region2 = (1/2π) ∫dV' [μ2Jc(x')] ln(1/R)     +    d   =  W + d

where constant d is a homo solution to (2 + β12) u  =  0 .

2. The solution in region 1 is a homo solution of this form:

AI,region1  = A lnr

3. I then evaluate the integral W. Just for fun, I do it for both region 1 and region 2, but I only need it right now for region 2 where r < a.

W = (1/2π) ∫dV' [μ2Jc(x')] ln(1/R)   = -(1/4π) μ2∫dV'[Jcz(x')] ln(R2) }  = -(1/4π) μ2Jcz∫dV' ln(R2) }

= -(1/4π) μ2[I/πa2] 	// using ∫dV' ln(R2) from App B

= -(1/4π) μ2[I/πa2] 2πa2

= -(I/2π) μ2

The relevant part is in region 2 where I use the integral above, where then

=  -(I/2π) μ2 [ lna + (1/2)(r2/a2-1) ] 			// integral evaluated in region 2

4. Here then is what the solutions look like in the two regions at this point:

AI,region1(r)  = A lnr

AI,region2(r) =  d  -  (I/2π) μ2 [ lna + (1/2)(r2/a2-1) ]

Notice these facts which will be used in the next little section

AI,region1(a)  = A lna

AI,region2(a) = d  -  (I/2π) μ2 lna

∂r AI,region1(r) = A/r

∂r AI,region2(r) =  -  (I/2π) μ2 * (1/2a2)2r   =   -  (I/2πa2) μ2 r

∂r AI,region1(a) = A/a

∂r AI,region2(a)   =   -  (I/2πa2) μ2 a

4. I then match the boundary conditions

AI,region2(a)  = AI,region1(a)

(1/μ2) ∂rAI,region2(a) = (1/μ1) ∂rAI,region1(a)

which conditions then say

d  -  (I/2π) μ2 lna  = A lna

(1/μ2) [-  (I/2πa2) μ2 a]  = (1/μ1)[ A/a]

or

d  -  (I/2π) μ2 lna  = A lna

-  (I/2π)   = (1/μ1)A

The solution to these 2 equations in 2 unknowns is

A = -μ1 (I/2π)

d = (I/2π) μ2 lna  -μ1 (I/2π) lna = (I/2π)(μ2-μ1)lna

Installing these constants, we then obtain our final solution

AI,region2(r) = W+d =  -(I/2π) μ2 [ lna + (1/2)(r2/a2-1) ] + (I/2π)(μ2-μ1)lna

AI,region1(r) = Alnr =  -μ1 (I/2π)lnr

or

AI,region2(r)  = -(I/2π) μ2 lna  -(I/4π) μ2(r2/a2-1) + (I/2π)(μ2-μ1)lna

AI,region1(r) = -μ1 (I/2π)lnr

or

AI,region2(r)  = -(I/2π) μ1 lna  - (I/4π) μ2(r2/a2-1)

AI,region1(r) = -μ1 (I/2π)lnr

or

AI,region2(r)  = -(I/2π) [  μ1 lna  +  (1/2)μ2(r2/a2-1)

AI,region1(r) = -μ1 (I/2π)lnr

Comment:  This is the problem solution obtained by treating each region separately and making assumptions about the assumed forms then finding the constants. [ By the way, this solution happens to agree with the Appendix B Version 2 solution. ]

Problem II Way.  [ this method works, but is not supportable] The problem is described uniformly for region 1 + 2 = region R, but an image surface current is added at the boundary whose value matches the known mag current there.

(2 + β12) AII  =   -μ0Jm  - μ2Jc2		x in region R		// image current present!

AII,region2(a)  = AII,region1(a)

(1/μ2) ∂rAII,region2(a) -  (1/μ1) ∂rAII,region1(a) = Kzimage =  [(μ1-μ2)/μ0] I/(2πa)   (*) wrong!

Question:  Below do I ever make use of equation (*) ?

Answer: No, I never use this, and moreover, it is not true! [ correct]  See comments in Plan C of Disaster Log 2.

To solve this problem I make these assumptions:

1. The entire solution is the particular solution of the uniform PDE

AII,regionR  = (1/2π) ∫dV' [μ2Jc(x') + μ0Jm(x')] ln(1/R)   = W + Z

where W is the same integral we saw in the Problem I analysis and which we happened to evaluate in both regions 1 and 2, to wit,

W = -(I/2π) μ2  .

The new integral here is then

Z =  (1/2π) ∫dV'Jzm(x')] ln(1/R)   = (1/2π) μ0 ∫dS' Kzm(x') ln(1/R)

=  (1/2π) μ0 [(μ1-μ2)/μ0] I/(2πa)  ∫dS' ln(1/R)

=  (1/2π)  (μ1-μ2) I/(2πa)  ∫dS' ln(1/R)

=  - (1/2π)  (μ1-μ2) I/(2πa)(1/2) { ∫dS' ln(R2) }

=  - (1/2π)  (μ1-μ2) I/(2πa)(1/2) 4πa 	// {..} from Appendix B

=  - (I/2π) (μ1-μ2)

=  - (I/2π) (μ1-μ2)

Then the unified solution over R is this

AII,regionR  = W + Z

=  -(I/2π) μ2   - (I/2π) (μ1-μ2)

= -(I/2π) [     μ2  + (μ1-μ2)     ]

We then write out the two region solutions separately

AII,regionR (r>a) =  -(I/2π)  [ μ2lnr + (μ1-μ2)lnr ] =   -(I/2π) μ1lnr

AII,regionR (r<a) = -(I/2π)[ μ2(lna + (1/2)(r2/a2-1)) + (μ1-μ2)lna ]

= -(I/2π)[ μ2lna + μ2 (1/2)(r2/a2-1) + (μ1-μ2)lna ]

= -(I/2π)[ μ1lna + μ2 (1/2)(r2/a2-1) ]

and once again

AII,regionR (r>a) =  -(I/2π) μ1lnr					region 1

AII,regionR (r<a) = -(I/2π)[ μ1lna + μ2 (1/2)(r2/a2-1) ]		region 2

[ Note:  these agree with the "correct" solutions obtained in the Disaster Log 2  "Resume 11.21.13. ]

Thus, I at least have A METHOD of solving this problem and getting the correct answer!]

Comparison of the solutions done the two ways

AI,region1(r) = -μ1 (I/2π)lnr

AI,region2(r)  = -(I/2π) [  μ1 lna  +  (1/2)μ2(r2/a2-1)

AII,region1(r) = -μ1 (I/2π)lnr

AII,region2(r)  = -(I/2π)[ μ1lna + μ2 (1/2)(r2/a2-1) ]

We observe that the solution to this problem is EXACTLY THE SAME using either method of computation:

I. Smythian form segregated method in each region. Simple homo form is selected for region 1, and a simple homo form constant adder is selected for region 2, and these are sufficient to solve the problem. Each region has a homo contribution.

II. Unified region R solution but an image current has been added at the boundary. There is no homo contribution at all.

This is my Prototype Example of the two Ways.

The question is:  WHY do these two Ways give the exact same answer? Is it just a coincidence for this very simple geometry, or is it a more general fact? [ I cannot prove it as general fact, so "cloy" ]

Comment: At r = 0 we get

AII,region2(0)  = -(I/2π)[ μ1lna + μ2 (1/2)( -1) ]	  =  -(I/2π)[ μ1lna - μ2/2 ]

AII,region2(a)  = -(I/2π)[ μ1lna + μ2 (1/2)(12-1) ]	  =  -(I/2π)[ μ1lna ]

]

Problem III Way ?

Blue shows the original method

curl H = ∂tD + J				// Maxwell (1.1.1)

(1/μ) curl curl A = με ∂tE + J		// H = B/μ , B = curl A from (1.3.1), and D = εE

grad divA - 2A  = με ∂t[- grad φ - ∂tA] + μJ	// vector identity and E = - grad φ - ∂tA

(2 - με ∂t2) A  = grad [με ∂tφ + divA ] - μJ	[ = Jackson (6.11) ] 			(1.3.3)

(2 - με ∂t2)A  = - μJ  .			// apply same gauge choice divA = - με ∂tφ

Black shows a possible different path:

curl B = μ0(∂tD + Jc + Jm)					// Maxwell (1.1.24)

grad divA - 2A  = μ0(∂tεE + Jc + Jm)

grad divA - 2A  = μ0(ε∂t[- grad φ - ∂tA] + Jc + Jm)

grad divA - 2A  = - μ0ε ∂tgrad φ - μ0ε∂t2A + μ0Jc + μ0Jm

- grad divA + 2A  =  μ0ε ∂tgrad φ + μ0ε∂t2A - μ0Jc - μ0Jm

2A - μ0ε∂t2A  =  μ0ε ∂tgrad φ + grad divA  - μ0Jc - μ0Jm

(2A - μ0ε∂t2) A  = grad [divA + μ0ε∂tφ]  - μ0Jc - μ0Jm

Still have not made gauge choice, but I can see that this is NOT the wave operator I want to see. This is totally at odds with the King wave operator, so this Way III is just not useful. Here I am trying to "expose" the mag current so it gets picked up. [ I pursue this more in Disaster 2, Plan E1. It led I think to me finding my big bug, but it is a lousy gauge to work in.  ]